DEPARTMENT: WATER AND SANITATION

CHIEF DIRECTORATE WATER ECOSYSTEMS

DIRECTORATE RESERVE DETERMINATION

DETERMINATION, REVIEW AND IMPLEMENTATION

OF THE RESERVE IN THE OLIFANTS/LETABA SYSTEM

QUANTIFICATION OF THE ECOLOGICAL WATER REQUIREMENTS

DRAFT

June 2016

DEPARTMENT: WATER AND SANITATION

Directorate: Reserve Determination

DETERMINATION, REVIEW AND IMPLEMENTATION OF THE RESERVE IN THE OLIFANTS/LETABA SYSTEM

WP10940

QUANTIFICATION OF ECOLOGICAL WATER REQUIREMENTS

JUNE 2016

REFERENCE

This report is to be cited as:

Chief Directorate:Water Ecosystems. Department of Water and Sanitation, South Africa, June 2016. DETERMINATION, REVIEW AND IMPLEMENTATION OF THE RESERVE IN THE OLIFANTS/LETABA SYSTEM: Quantification of Ecological Water Requirements Report. Report No:RDM/WMA02/00/CON/0216

Prepared by:

Golder Associates Africa, Wetland Consulting Services, JMM Stassen and WRP Consulting Engineers

Title:	Quantification of Ecological Water Requirements Report

Authors:	Retha Stassen, Wynand Vlok, Kylie Farrell, Trevor Coleman, Priya Moodley

Project Name:	Determination, Review and Implementation of the Reserve in the Olifants/Letaba System: WP 10940

DWA Report No:	RDM/WMA02/00/CON/0216

Status of Report:	Draft

First Issue:	June 2016

Final Issue:

Professional Service Providers:Golder Associates Africa/Wetland Consulting Services/ JMM Stassen and WRP Consulting Engineers

Approved for the PSP by:

…………………………………………………

# Trevor Coleman

# Study Leader

DEPARTMENT OF WATER AND SANITATION

Chief Directorate: Water Ecosystems

Approved for DWS by:

# ………………………………………

# Yakeen Atwaru

# Director: Reserve Determination

…………………………………………………

# Ndileka Mohapi

# Chief Director: Water Ecosystems

DOCUMENT INDEX

Reports as part of this project:

Bold type indicates this report.

| Report INDEX | REPORT NUMBER | Report title |
| --- | --- | --- |
| 1.0 | RDM/WMA02/00/CON/0115 | Inception Report |
| 2.0 | RDM/WMA02/00/CON/0215 | Information and Data Gap Analysis Report |
| 3.0 | RDM/WMA02/00/CON/0315 | Field Survey Report |
| 4.0 | RDM/WMA02/00/CON/0116 | Eco-Classification Report |
| 5.0 | RDM/WMA02/00/CON/0216 | Quantification of Ecological Water Requirements Report |

LIST OF ABBREVIATIONS

| # BHN | # Basic Human Needs |
| --- | --- |
| # DRM | # Desktop Reserve Model |
| # DWS | # Department of Water and Sanitation |
| # EI | # Ecological Importance |
| # EIS | # Ecological Importance and Sensitivity |
| # ES | # Ecological Sensitivity |
| # EWR | # Ecological Water Requirements |
| # IWRMP | # Integrated Water Resource Management Plan |
| # KNP | # Kruger National Park |
| # MAR | # Mean Annual Runoff |
| # NWA | # National Water Act |
| # PES | # Present Ecological State |
| # RDM | # Resource Directed Measures |
| # REC | # Recommended Ecological Category |
| # RQO | # Resource Quality Objectives |
| # WMA | # Water Management Area |
| # WRCS | # Water Resources Classification System |

EXECUTIVE SUMMARY

The Chief Directorate: Water Ecosystems commissioned the study ‘Determination, Review and Implementation of the Reserve in the Olifants/Letaba System’ in 2015. The purpose of this study is to determine, review and implement the Reserve in the Olifants/Letaba Catchments, with the aim of specifically addressing ecological gaps and reviewing the preliminary Reserves that have been determined.

Previous relevant studies completed for these systems are:

- Determination of the preliminary Reserve for the Olifants System (2001) and for the Letaba system (2006); and

- Classification and determination of Resource Quality Objectives (RQOs) for the water resources of the Olifants River catchment (2011-2013) and for the water resources of the Letaba River catchment (2012-2014).

As the classes of the water resources for the Olifants/Letaba systems have now been determined, the preliminary Reserve can be superseded with The Reserve and gazetted.

Four main components are being addressed through this study following the 8 step Reserve determination procedure, namely:

- The review and analysis of existing information;

- Identification and filling in of the ecological gaps identified;

- Evaluation of ecological consequences and operational considerations; and

- Setting the Reserve, defining the ecological specifications and developing the resource management plan.

The review and analysis of existing information, identification of ecological gaps and the filling in of the ecological gaps through Rapid III Reserve determinations and biological surveys of the priority sites identified in the Olifants, Letaba and Shingwedzi catchments have been completed. The priority sites selected for this study included the following:

- New Ecological Water Requirement (EWR) sites where no or little information were available;

- Re-surveying of existing EWR sites where no new information after the initial preliminary Reserves in 2001 and 2006 were available; and

- The re-assessment of existing EWR sites using the hydraulic information from the previous Reserve studies with updated biological information.

This report provides the results of step 4 of the 8-step Reserve determination process, the quantification of the Ecological Water Requirements (EWR), based on the Rapid III level of detail for the Priority Rivers as identified for the Olifants Water Management Area.

The results and findings of the quantification of the EWR for the identified sites are summarised below for the Olifants and the Letaba/Shingwedzi catchments respectively.

Olifants catchment

| Quaternary Catchment | River | Recommended Ecological Category | MAR (106m3) | Total EWR as %MAR | Overall confidence |
| --- | --- | --- | --- | --- | --- |
| B31C | Upper Elands | C | 31.075 | 20.87 | Low |
| B20J | Lower Wilge | B | 175.58 | 36.28 | Low to moderate |
| B11J | Olifants | C/D | 184.53 | 18.42 | Low to moderate |
| B12D | Klein Olifants | C/D | 66.649 | 27.47 | Low to moderate |
| B32A | Olifants | B/C | 500.63 | 29.83 | Low to moderate |
| B32A | Kranspoortspruit | B | 4.712 | 41.16 | Low to moderate |
| B32C | Selons | C | 33.109 | 21.86 | Low |
| B71D | Olifants | C | 813.17 | 20.87 | Low to moderate |
| B42H | Lower Spekboom | B/C | 148.19 | 30.79 | Low |
| B60B | Upper Blyde | B | 164.45 | 46.08 | Low to moderate |
| B71J | Olifants | C | 1 321.92 | 17.85 | Low to moderate |
| B60J | Lower Blyde | B | 363.27 | 32.87 | Low to moderate |
| B72D | Olifants | B/C | 1 762.1 | 29.94 | Low to moderate |
| B73H | Olifants | B/C | 1 918.3 | 29.54 | Low to moderate |
| B32D | Olifants | C | 571.13 | 12.51 | Low to moderate |
| B31G | Lower Elands | C/D | 60.32 | 10.77 | Low |
| B51G | Olifants | D | 726.64 | 16.14 | Low |
| B41H | Steelpoort | C | 137.5 | 27.39 | Low to moderate |

Letaba and Shingwedzi catchments

| Quaternary Catchment | River | Recommended Ecological Category | MAR (106m3) | Total EWR as %MAR | Overall confidence |
| --- | --- | --- | --- | --- | --- |
| B83D | Letaba | C | 646.28 | 17.34 | Low to moderate |
| B81D | Letsitele | D | 116.55 | 17.59 | Low to moderate |
| B81B | Great Letaba | C | 99.85 | 24.76 | Low to moderate |
| B81A | Broederstroom | B/C | 6.683 | 49.22 | Low |
| B90H | Shingwedzi | B/C | 86.424 | 22.5 | Low |

The present state of most of the tributaries, as well as the main stem of the Olifants, Letaba and Shingwedzi Rivers, have been degraded over the past few years since the previous comprehensive Reserve studies conducted in 2001 and 2006. This can also be attributed to the environmental stresses on the aquatic ecosystem, owing to the severe drought conditions which occurred during the surveys in October 2015 and April 2016.

Some of the sites visited were either dry or had very low flows that limited the habitat availability for fish and macroinvertebrates. The low flows further contributed to poor water quality conditions with limited dilution capacity. The poor water quality in the Olifants catchment is a result of the extensive mining activities in especially the upper catchments, agricultural activities and urbanisation. Extensive irrigation and urbanisation in the Letaba catchment contribute to elevated levels of Total Dissolved Solids (TDS) and observed algal blooms (nutrients). Sites within the Letaba catchment upstream of the KNP were further impacted by alien invasive species, both aquatic and terrestrial, thus changing the indigenous biotic composition of the river and consequently lowering the PES.

Water use outside the KNP is impacting on the already non-perennial rivers of the Shingwedzi catchment, resulting in longer periods of zero flows.

Thus, the implementation of the EWRs as specified in this report, together with ongoing monitoring will assist in the sustainable management and conservation of the water resources of the Olifants, Letaba and Shingwedzi catchments.

TABLE OF CONTENTS

1.	INTRODUCTION	1

1.1	background and context	1

1.2	Purpose of the study	2

1.3	study area and ewr sites	3

1.4	Approach	7

2.	DATA COLLECTION AND MODELLING	10

2.1	Hydraulics	10

2.2	Hydrological data	14

2.3	Quantification of EWRs	15

2.4	Water Quality	16

2.4.1	APPROACH	16

3.	RESULTS (QUANTITY)	18

3.1	Olifants_S1: Upper Elands River in B31C	18

3.2	Olifants_S2: Lower Wilge River in B20J	19

3.3	Olifants_S3: Upper Wilge River in B20F	22

3.4	Olifants_S5: Olifants River in B11J	25

3.5	Olifants_S6: Klein Olifants River in B12D	28

3.6	Olifants_S7: Olifants River in B32A	31

3.7	Olifants_S8: Kranspoortspruit in B32A	34

3.8	Olifants_S9: Selons River in B32C	36

3.9	Olifants_S10: Olifants River in B71D	39

3.10	Olifants_S11: Lower Spekboom River in B42H	42

3.11	Olifants_S12: Upper Blyde River in B60B	44

3.12	Olifants_S13: Olifants River in B71J	48

3.13	Olifants_S14: Lower Blyde River in B60J	51

3.14	Olifants_S15: Olifants River in B72D	54

3.15	Olifants_S16: Olifants River in B73H	56

3.16	Olifants_EWR5: Olifants River in B32D	59

3.17	Olifants_EWR6: Lower Elands River in B31G	60

3.18	Olifants_EWR7: Olifants River in B51G	62

3.19	Olifants_EWR9: Steelpoort River in B41H	63

3.20	LET2: Letaba River in B83D	65

3.21	LET14: Letsitele River in B81D	68

3.22	LET16: Great Letaba River in B81B	71

3.23	LET18: Broederstroom in B81A	74

3.24	SHI1: ShingwedzI River in B90H	77

4.	RESULTS (QUALITY)	79

5.	CONCLUSIONS	84

6.	REFERENCES	86

LIST OF TABLES

Table 1: Summary of EWR sites in Olifants, Letaba and Shingwedzi catchments assessed during the implementation study	8

Table 2: Hydraulics data measured for the Olifants, Letaba and Shingwedzi EWR sites	10

Table 3: Hydraulic data used to extend observed rating data at the EWR sites	11

Table 4: Regression coefficients in equation (1)	12

Table 5: Confidence in the hydraulic modelled results	13

Table 6: Natural MAR per EWR site in the Olifants, Letaba and Shingwedzi catchments	14

Table 7: Summary of Resource Quality Objectives for Water Quality (Rivers) for the Olifants River system	17

Table 8: Summary of the final EWR results (flows in million m3 per annum)	19

Table 9: Freshets and floods for the 2001 study and final requirements for implementation	21

Table 10: EWR results for specific months for the Lower Wilge River in B20J (REC = B)	21

Table 11: Summary of the final EWR results (flows in million m3 per annum)	22

Table 12: Freshets and floods for the Upper Wilge EWR site in B20F	24

Table 13: EWR results for specific months for the Upper Wilge River in B20F (REC = C)	24

Table 14: Summary of the final EWR results (flows in million m3 per annum)	24

Table 15: Freshets and floods for the 2001 study and final requirements for implementation	27

Table 16: EWR results for specific months for the Olifants River in B11J (REC = C/D)	27

Table 17: Summary of the final EWR results (flows in million m3 per annum)	27

Table 18: Freshets and floods for the Klein Olifants EWR site in B12D	30

Table 19: EWR results for specific months for the Klein Olifants River in B12D (REC = C/D)	30

Table 20: Summary of the final EWR results (flows in million m3 per annum)	30

Table 21: Freshets and floods for the 2001 study and final requirements for implementation	33

Table 22: EWR results for specific months for the Olifants River in B32A (REC = B/C)	33

Table 23: Summary of the final EWR results (flows in million m3 per annum)	33

Table 24: EWR results for specific months for the Kranspoortspruit in B32A (REC = B)	36

Table 25: Summary of the final EWR results (flows in million m3 per annum)	36

Table 26: EWR results for specific months for the Selons River in B32C (REC = C)	38

Table 27: Summary of the final EWR results (flows in million m3 per annum)	38

Table 28: Freshets and floods for the 2001 study and final requirements for implementation	41

Table 29: EWR results for specific months for the Olifants River in B71D (REC = C)	41

Table 30: Summary of the final EWR results (flows in million m3 per annum)	42

Table 31: EWR results for specific months for the Lower Spekboom River in B42H (REC = B/C)	44

Table 32: Summary of the final EWR results (flows in million m3 per annum)	44

Table 33: Freshets and floods requirements for implementation	46

Table 34: EWR results for specific months for the Upper Blyde River in B60B (REC = B)	47

Table 35: Summary of the final EWR results (flows in million m3 per annum)	47

Table 36: Freshets and floods for the 2001 study and final requirements for implementation	50

Table 37: EWR results for specific months for the Olifants River in B71J (REC = C)	50

Table 38: Summary of the final EWR results (flows in million m3 per annum)	51

Table 39: Freshets and floods for the 2001 study and final requirements for implementation	53

Table 40: EWR results for specific months for the Lower Blyde River in B60J (REC = B)	53

Table 41: Summary of the final EWR results (flows in million m3 per annum)	54

Table 42: Freshets and floods for the 2001 study and final requirements for implementation	55

Table 43: Summary of the final EWR results (flows in million m3 per annum)	56

Table 44: Freshets and floods for the 2001 study and final requirements for implementation	58

Table 45: EWR results for specific months for the Olifants River in B73H (REC = B/C)	58

Table 46: Summary of the final EWR results (flows in million m3 per annum)	58

Table 47: Recommended flows for EWR5 on the Olifants River in B32D (REC=C)	59

Table 48: Summary of the final EWR results (flows in million m3 per annum)	60

Table 49: Recommended flows for EWR6 on the Elands River in B31G (REC=C/D)	61

Table 50: Summary of the final EWR results (flows in million m3 per annum)	62

Table 51: Recommended flows for EWR7 on the Olifants River in B51G (REC=D)	62

Table 52: Summary of the final EWR results (flows in million m3 per annum)	63

Table 53: Recommended freshets/ floods for EWR9 on the Steelpoort River in B41H	64

Table 54: Summary of the final EWR results (flows in million m3 per annum)	65

Table 55: Freshets and floods for the 2013 study and final requirements for implementation	67

Table 56: EWR results for specific months for the Letaba River in B83D (REC = C)	67

Table 57: Summary of the final EWR results (flows in million m3 per annum)	67

Table 58: Freshets and floods for the 2013 study and final requirements for implementation	70

Table 59: EWR results for specific months for the Letsitele River in B81D (REC = D)	70

Table 60: Summary of the final EWR results (flows in million m3 per annum)	71

Table 61: Freshets and floods for the 2013 study and final requirements for implementation	73

Table 62: EWR results for specific months for the Great Letaba River in B81B (REC = C)	73

Table 63: Summary of the final EWR results (flows in million m3 per annum)	74

Table 64: Freshets and floods requirements for implementation for the Broederstroom in B81A	76

Table 65: EWR results for specific months for the Broederstroom in B81A (REC = B/C)	76

Table 66: Summary of the final EWR results (flows in million m3 per annum)	77

Table 67: EWR results for specific months for the Shingwedzi River in B90H (REC = B/C)	79

Table 68: Summary of the final EWR results (flows in million m3 per annum)	79

Table 69: Overview of water quality state and associated land use impacts in the rivers in the Olifants/Letaba System	80

LIST OF FIGURES

Figure 1: Map of the Olifants Catchment illustrating the 1999 EWR sites, re-assessed EWR sites and the biological sites	5

Figure 2: Map of the Letaba and Shingwedzi catchment area illustrating the 1999 EWR sites, re-assessed EWR sites and the biological sites	6

Figure 3: View of the EWR site on the Upper Elands River	18

Figure 4: View of the EWR site on the Lower Wilge River	19

Figure 5: Water levels on cross-section of the EWR site for the Lower Wilge River in B20J	20

Figure 6: View of the EWR site on the Upper Wilge River	22

Figure 7: Water levels on cross-section of the EWR site for the Upper Wilge River in B20F	23

Figure 9: View of the EWR site on the Olifants River	25

Figure 10: Water levels on cross-section of the EWR site for the Olifants River in B11J	26

Figure 11: View of the EWR site on the Klein Olifants River	28

Figure 12: Water levels on cross-section of the EWR site for the Klein Olifants River in B12D	29

Figure 13: View of the EWR site on the Olifants River in B32A	31

Figure 14: Water levels on cross-section of the EWR site for the Olifants River in B32A	32

Figure 15: View of the EWR site on the Kranspoortspruit	34

Figure 16: Water levels on cross-section of the EWR site for the Kranspoortspruit in B32A	35

Figure 17: View of the EWR site on the Selons River	37

Figure 18: Water levels on cross-section of the EWR site for the Selons River in B32C	37

Figure 19: View of the EWR site on the Olifants in B71D	39

Figure 20: Water levels on cross-section of the EWR site for the Olifants River in B71D	40

Figure 21: View of the EWR site on the Lower Spekboom River	42

Figure 22: Water levels on cross-section of the EWR site for the Lower Spekboom River in B42H	43

Figure 23: View of the EWR site on the Upper Blyde River	45

Figure 24: Water levels on cross-section of the EWR site for the Upper Blyde River in B60B	46

Figure 25: View of the EWR site on the Olifants River in B71J	48

Figure 26: Water levels on cross-section of the EWR site for the Olifants River in B71J	49

Figure 27: View of the EWR site on the Lower Blyde River	51

Figure 28: Water levels on cross-section of the EWR site for the Lower Blyde River in B60J	52

Figure 29: View of the EWR site on the Olifants River in B72D	54

Figure 30: View of the EWR site on the Olifants River in B73H	56

Figure 31: Water levels on cross-section of the EWR site for the Olifants River in B73H	57

Figure 32: Water levels on cross-section of Olifants_EWR site for the Olifants River in B32D	60

Figure 33: Water levels on cross-section of Olifants_EWR6 site for the Elands River in B31G	61

Figure 34: Water levels on cross-section of Olifants_EWR7 site for the Olifants River in BB51G	63

Figure 35: Water levels on cross-section of Steelpoort River for Olifants_EWR9 site in B41H	64

Figure 36: View of the EWR site on the Letaba River in B83D	65

Figure 37: Water levels on cross-section of the EWR site for the Letaba River in B83D	66

Figure 38: View of the EWR site on the Letsitele River	68

Figure 39: Water levels on cross-section of the EWR site for the Letsitele River in B81D	69

Figure 40: View of the EWR site on the Great Letaba River in B81B	71

Figure 41: Water levels on cross-section of the EWR site for the Great Letaba River in B81B	72

Figure 42: View of the EWR site on the Broederstroom	74

Figure 43: Water levels on cross-section of the EWR site for the Broederstroom in B81A	75

Figure 44: View of the EWR site on the Shingwedzi River	77

Figure 45: Water levels on cross-section of the EWR site for the Shingwedzi River in B90H	78

ELECTRONIC APPENDICES

APPENDIX A		HYDRAULIC RESULTS

APPENDIX B		NATURAL FLOWS FOR EWR SITES

APPENDIX C		DRM RESULTS

APPENDIX D		EWR SITE PHOTOS

# INTRODUCTION

## background and context

With the promulgation of The National Water Act (No. 36 of 1998) (NWA), water resources management in South Africa underwent a paradigm shift. The Department of Water and Sanitation (DWS) as custodian of the nation’s water resources is mandated to protect, use, develop, conserve, manage and control the nation’s water resources in a sustainable and equitable manner for the benefit of all South Africans.

Chapter 3 of the NWAwas specifically developed for the protection of the water resources of the country and requires the implementation of Resource Directed Measures (RDM) that is based on the guiding principles of sustainability and equity. Sustainability encompasses both the long- and short-term protection of water resources to ensure that they can be developed and used effectively into the future.

The RDM,as enabled through the NWA, provides for the protection of water resources through (1) the Classification of water resources, (2) determination of Resource Quality Objectives (RQOs) and (3) the determination of the Reserve. These measures collectively aim to ensure that a balance is reached between the need to protect and sustain water resources on one hand and the need to develop and use them on the other.

- Water Resources Classification System: The NWA makes provision in section 12 for the development of a national classification system for the classification of all significant water resources. The management class of a water resource as determined by the classification system is based on ecological, social and economic considerations.

- Resource Quality Objectives (RQOs): These are defined as clear goals (numerical or descriptive statements) relating to the quality of a water resource and are set in accordance to the management class specified for the resource to ensure the water resource is protected. The purpose of RQOs is to set clear objectives for the resource against which water use licenses and the related impacts can be evaluated and managed to achieve a balance between the need to protect and utilization of the resource.

- The Reserve: Specifies the quantity, quality, habitat and biotic integrity requirements necessary for the protection of the resource and has priority over other water uses. Two components are provided for, namely:

- Basic human needs (BHN), ensuring that the essential needs of individuals served by the water resource in question are provided for; and

- The ecological Reserve ensuring the water required to protect aquatic ecosystems of the water resource are provided for.

The ecological Reserve is not intended to protect the aquatic ecosystem per se, but to maintain aquatic ecosystems in such a way that they can continue to provide the goods and services to society and is specified for groundwater, wetlands, rivers and estuaries.

## Purpose of the study

The Chief Directorate: Water Ecosystems in 2015 commissioned the study ‘Determination, Review and Implementation of the Reserve in the Olifants/Letaba System.  The purpose of this study is thus to determine, review and implement the Reserve in the Olifants\Letaba System; with the aim of specifically addressing ecological gaps and reviewing and refining the preliminary Reserves that have been determined during previous studies.

Previous relevant studies completed for these systems are:

- Determination of the preliminary Reserve for the Olifants System (2001) and for the Letaba system (2006); and

- Classification and determination of RQOs for the water resources of the Olifants River catchment (2011-2013 and for the water resources of the Letaba River catchment (2012-2014);

As the class of the water resources for the Olifants/Letaba systems have now been determined, the preliminary Reserve can now be superseded with The Reserve and gazetted.

Due to the preliminary Reserve having been determined in 2001 and 2006 prior to the water resource classification in 2011-2014, a review and update is required to ensure that the Reserve is in accordance with the water resource classes and is applicable to the current system needs and demands. Four main components are being addressed through the study technical process, namely:

- The review and analysis of existing information;

- Identification and filling in of the ecological gaps identified;

- Evaluation of ecological consequences and operational considerations; and

- Setting the Reserve, defining the ecological specifications and developing the resource management plan.

The 8 step Reserve determination procedure is being adhered to in terms of addressing the ecological gaps.

The review and analysis of existing information, identification of ecological gaps and the filling in of the ecological gaps through Rapid III Reserve determinations and biological surveys of the priority sites identified in the Olifants, Letaba and Shingwedzi catchments have been completed. The priority sites selected for this study included the following:

- New EWR sites where no or little information were available;

- Re-surveying of existing EWR sites where no new information after the initial preliminary Reserves in 2001 and 2006 were available; and

- The re-assessment of existing EWR sites using the hydraulic information from the previous Reserve studies with updated biological information.

This report provides the results of step 4 of the 8-step Reserve determination process, the quantification of the Ecological Water Requirements (EWR), based on the Rapid III level of detail for the priorityrivers as identified forthe Olifants Water Management Area.

The results of the gap analysis and Ecoclassification process is provided in separate reports.

## study area and ewr sites

The study area is the Olifants WMA (WMA 2) and includes the Olifants, Letaba and Shingwedzi River systems. The spatial extent of the area includes secondary drainage regions B1-B7 (Olifants), B8 (Letaba) and B9 (Shingwedzi).

The Olifants River originates at Trichardt, east of Johannesburg, and flows through the Kruger National Park. The Letaba River joins the Olifants River upstream of the border into Mozambique, where they join the Limpopo River before discharging into the Indian Ocean. The Shingwedzi River is located mostly in the Kruger National Park and then into Mozambique before it joins the Limpopo River.

The Olifants System falls within three provinces (Gauteng, Mpumalanga and the Limpopo Province). The main tributaries of the Olifants River are the Wilge, Elands and Ga-Selati Rivers on the left bank and the Klein-Olifants, Steelpoort, Blyde, Klasserie and Timbavati Rivers on the right bank.

The Letaba River catchment is drained by the Groot Letaba River and its major tributaries are the Klein-Letaba, Middle Letaba, Letsitele and Molototsi Rivers. The Shingwedzi River and its major tributaries the Shisha, Mphongolo and Phugwane drain the Shingwedzi River catchment.

The Olifants WMA is a highly utilised and regulated catchment and like many other WMAs in South Africa, its water resources are becoming more stressed due to an accelerated rate of development and the scarcity of water resources. There is an urgency to ensure that water resources in the Olifants WMA are able to sustain their level of uses and be maintained at their desired ecological states.

Figure 1and Figure 2shows the catchment areas of the Olifants WMA and the various EWR sites, including those from previous studies that were re-assessed during this study coupled with the biological sites.

Figure 1: Map of the Olifants Catchment illustrating the 1999 EWR sites, re-assessed EWR sites and the biological sites

Figure 2: Map of the Letaba and Shingwedzi catchment area illustrating the 1999 EWR sites, re-assessed EWR sites and the biological sites

## Approach

The quantification of the ecological water requirements were determined using the following:

- Information collected during the field surveys;

- Results from the Eco-classification process (Present Ecological State (PES), Ecological Importance(EI), Ecological Sensitivity (ES) and Recommended Ecological Category (REC));

- Desktop Reserve Model (DRM) within SPATSIM for the integration of data produced from the surveys and Eco-classification to quantify the EWRs;

- Results from the hydraulic modelling (cross-sectional profile and discharge) to evaluate the DRM requirements; and

- Evaluation of the water quality at specific selected sites where quality was identified as an issue.

Table 1provides a summary of the information for all the EWR sites where the EWRs were quantified or re-assessed.

Table 1: Summary of EWR sites in Olifants, Letaba and Shingwedzi catchments assessed during the implementation study

| EWR site | Quaternary catchment/ Sub-reach | River | Coordinates | PES | EI_ES | REC | Level andcomments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Olifants Catchment |  |  |  |  |  |  |  |
| Olifants_S1 | B31C-00770 | Upper Elands | -25.3031; 28.4631 | C/D | Very high | C | New rapid 2 (no hydraulics) |
| Olifants_S2 | B20J-00998 | Lower Wilge | -25.6196;28.9991 | C | High | B | Replace existing Olifants_EWR4, rapid 3 |
| Olifants_S3 | B20F-01150 | Upper Wilge | -25.8439;28.8719 | C/D | High | C | New rapid 3 |
| Olifants_S5 | B11J-01086 | Olifants | -25.7592;29.3096 | D | Moderate | C/D | Replace existing Olifants_EWR1, rapid 3 |
| Olifants_S6 | B12D-01118 | Klein Olifants | -25.7489;29.4587 | D/E | High | C/D | New rapid 3 |
| Olifants_S7 | B32A-00937 | Olifants | -25.4963;29.2546 | C | High | B/C | Replace existing Olifants_EWR2, rapid 3 |
| Olifants_S8 | B32A-00950 | Kranspoortspruit | -25.4377;29.4756 | C | Very high | B | Resurvey existing OLI-EWR3 |
| Olifants_S9 | B32C-00936 | Selons | -25.3799;29.4356 | D | Very high | C | New rapid 3 |
| Olifants_S10 | B71D-00412 | Olifants | -24.2399; 30.0825 | C | Moderate | C | Replace existing Olifants_EWR8, rapid 3 |
| Olifants_S11 | B42H-00553 | Lower Spekboom | -24.6942; 30.3613 | C | High | B/C | New rapid 3 |
| Olifants_S12 | B60B-00566 | Upper Blyde | -24.7344; 30.7783 | C | High | B | New rapid 3 |
| Olifants_S13 | B71G-00428 | Olifants | -24.3076; 30.7857 | C | High | C | Replace existing Olifants_EWR11, rapid 3 |
| Olifants_S14 | B60J-00444 | Lower Blyde | -24.4075; 30.8274 | C | High | B | Replace existing Olifants_EWR12, rapid 3 |
| Olifants_S15 | B72D-00326 | Olifants | -24.1284; 31.0146 | C | Moderate | B/C | Replace existing Olifants_EWR13, rapid 2 (no hydraulics) |
| Olifants_S16 | B73H-00311 | Olifants | -24.0494; 31.7318 | D | High | B/C | Replace existing Olifants_EWR16, rapid 3 |
| Olifants_EWR5 | B32D-00855 | Olifants | -25.3040; 29.4220 | C | High | C | Use existing information and re-evaluate EWR |
| Olifants_EWR6 | B31F-00654 | Elands | -25.1160; 28.9565 | E | Moderate | D | Use existing information and re-evaluate EWR |
| Olifants_EWR7 | B51G-00482 | Olifants | -24.5289; 29.5464 | E | Moderate | D | Use existing information and re-evaluate EWR |
| Olifants_EWR9 | B41H-00610 | Steelpoort | -24.7750; 30.1650 | D | High | C/D | Use existing information and re-evaluate EWR |
| Letaba Catchment |  |  |  |  |  |  |  |
| LET2 | B83D-00250 | Letaba | -23.8268; 31.5906 | C/D | High | C | Downstream existing Letaba_EWR7, rapid 3 |
| LET14 | B81D-00271 | Letsitele | -23.8932; 30.3576 | D | High | D | Replace existing Letaba_EWR2, rapid 3 |
| LET16 | B81B-00264 | Great Letaba | -23.9178; 30.0507 | C/D | High | C | Replace existing Letaba_EWR1, rapid 3 |
| LET18 | B81A-00242 | Broederstroom | -23.8011; 29.9772 | C | High | B/C | New rapid 3 |
| Shingwidzi catchment |  |  |  |  |  |  |  |
| SHI1 | B90H-00117 | Shingwedzi | -23.1849; 31.5251 | C | High | B/C | New rapid 3 |

# DATA COLLECTION AND MODELLING

## Hydraulics

During the site visit, the following activities were undertaken:

- EWR cross section was selected;

- A survey of the cross sectional profile of the site was carried out;

- Longitudinal water slope was surveyed;

- Discharge was measured;

- GPS co-ordinates of the site were captured; and

- EWR site photographs were taken.

The hydraulic data collected during the site visit is listed inTable 2.

Table 2: Hydraulics data measured for the Olifants, Letaba and Shingwedzi EWR sites

| EWR site | Survey date | River | Discharge Q (m3/s) | Maximum flow depth (m) |
| --- | --- | --- | --- | --- |
| Olifants catchment |  |  |  |  |
| Olifants_S1* | 07/10/2015 | Upper Elands | 0.009 | n/a |
| Olifants_S2 | 08/10/2015 | Lower Wilge | 0.125 | 0.267 |
| Olifants_S3 | 08/10/2015 | Upper Wilge | 0.004 | 0.048 |
| Olifants_S5 | 09/10/2015 | Olifants | 0.163 | 0.363 |
| Olifants_S6 | 09/10/2015 | Klein Olifants | 0.391 | 0.291 |
| Olifants_S7 | 10/10/2015 | Olifants | 2.200 | 0.462 |
| Olifants_S8 | 11/10/2015 | Kranspoortspruit | 0.012 | 0.14 |
| Olifants_S9 | 11/10/2015 | Selons | 0.038 | 0.133 |
| Olifants_S10 | 12/10/2015 | Olifants | 2.343 | 0.451 |
| Olifants_S11 | 12/10/2015 | Lower Spekboom | 0.028 | 0.121 |
| Olifants_S12 | 13/10/2015 | Upper Blyde | 2.039 | 0.392 |
| Olifants_S13 | 14/10/2015 | Olifants | 4.051 | 0.541 |
| Olifants_S14 | 13/10/2015 | Lower Blyde | 0.653 | 0.303 |
| Olifants_S15* | 14/10/2015 | Olifants | 1.660 | n/a |
| Olifants_S16 | 15/10/2015 | Olifants | 2.964 | 0.513 |
| Letaba catchment |  |  |  |  |
| LET2 | 18/04/2016 | Letaba | 0.530 | 0.105 |
| LET14 | 21/04/2016 | Letsitele | 0.668 | 0.33 |
| LET16 | 22/04/2016 | Great Letaba | 0.747 | 0.455 |
| LET18 | 22/04/2016 | Broederstroom | 0.192 | 0.242 |
| Shingwedzi catchment |  |  |  |  |
| SHI1 | 19/04/2016 | Shingwedzi | 0.006 | 0.060 |

* No cross-sectional profile was surveyed due to multiple channels

Modelling was carried out using the measured data, as well as two modelled points to develop stage discharge curves.The following data was required in the use of the modelling: y (maximum flow depth), n (resistance coefficient), S (slope), Q (discharge), A (area) and WP (wetted perimeter). The measured and modelled data are shown in Table 3 below.

Table 3: Hydraulic data used to extend observed rating data at the EWR sites

| EWR site | River | Discharge Q (m3/s) | Maximum flow depth (m) | Manning’s resistance, n | Surface Slope, S m/m) | Ave. Velocity, V (m/s) |
| --- | --- | --- | --- | --- | --- | --- |
| Olifants catchment |  |  |  |  |  |  |
| Olifants_S2 | Lower Wilge | 0.125 | 0.267 | 0.329 | 0.02 | 0.091 |
| Olifants_S3 | Upper Wilge | 0.004 | 0.048 | 0.146 | 0.003 | 0.04 |
| Olifants_S5 | Olifants | 0.163 | 0.363 | 0.302 | 0.04 | 0.16 |
| Olifants_S6 | Klein Olifants | 0.391 | 0.291 | 0.112 | 0.0193 | 0.31 |
| Olifants_S7 | Olifants | 2.200 | 0.462 | 0.113 | 0.033 | 0.62 |
| Olifants_S8 | Kranspoortspruit | 0.012 | 0.14 | 1.048 | 0.01 | 0.015 |
| Olifants_S9 | Selons | 0.038 | 0.133 | 0.121 | 0.02 | 0.173 |
| Olifants_S10 | Olifants | 2.343 | 0.451 | 0.064 | 0.0038 | 0.37 |
| Olifants_S11 | Lower Spekboom | 0.028 | 0.121 | 0.351 | 0.04 | 0.09 |
| Olifants_S12 | Upper Blyde | 2.039 | 0.392 | 0.074 | 0.042 | 0.741 |
| Olifants_S13 | Olifants | 4.051 | 0.541 | 0.106 | 0.014 | 0.518 |
| Olifants_S14 | Lower Blyde | 0.653 | 0.303 | 0.156 | 0.036 | 0.338 |
| Olifants_S16 | Olifants | 2.964 | 0.513 | 0.041 | 0.0009 | 0.225 |
| Letaba catchment |  |  |  |  |  |  |
| LET2 | Letaba | 0.530 | 0.105 | 0.0084 | 0.003 | 0.298 |
| LET14 | Letsitele | 0.668 | 0.33 | 0.059 | 0.009 | 0.431 |
| LET16 | Great Letaba | 0.747 | 0.455 | 0.079 | 0.006 | 0.372 |
| LET18 | Broederstroom | 0.192 | 0.242 | 0.054 | 0.0125 | 0.549 |
| Shingwedzi catchment |  |  |  |  |  |  |
| SHI1 | Shingwedzi | 0.006 | 0.06 | 0.189 | 0.006 | 0.033 |

The depth/discharge relationship (Hirschowitz PM, Birkhead AL, James CS) was determined using the following equation:

y = aQb + c

Y is the maximum depth, Q is the discharge (m3/s) and a, b and c coefficients.The coefficients used in equation (1) are shown in Table 4 below.

Table 4: Regression coefficients in equation (1)

| EWR site | River | Regression coefficients |  |  |
| --- | --- | --- | --- | --- |
|  |  | a | b | c |
| Olifants catchment |  |  |  |  |
| Olifants_S2 | Lower Wilge | 0.4778 | 0.28 | 0 |
| Olifants_S3 | Upper Wilge | 0.4487 | 0.4089 | 0 |
| Olifants_S5 | Olifants | 0.6044 | 0.2814 | 0 |
| Olifants_S6 | Klein Olifants | 0.3936 | 0.296 | 0 |
| Olifants_S7 | Olifants | 0.3614 | 0.3164 | 0 |
| Olifants_S8 | Kranspoortspruit | 0.5269 | 0.3002 | 0 |
| Olifants_S9 | Selons | 0.4083 | 0.3476 | 0 |
| Olifants_S10 | Olifants | 0.3465 | 0.3158 |  |
| Olifants_S11 | Lower Spekboom | 0.4848 | 0.3905 | 0 |
| Olifants_S12 | Upper Blyde | 0.3303 | 0.2473 | 0 |
| Olifants_S13 | Olifants | 0.351 | 0.3112 | 0 |
| Olifants_S14 | Lower Blyde | 0.3446 | 0.2982 | 0 |
| Olifants_S16 | Olifants | 0.3837 | 0.2681 | 0 |
| Letaba catchment |  |  |  |  |
| LET2 | Letaba | 0.1371 | 0.4171 | 0 |
| LET14 | Letsitele | 0.3752 | 0.304 | 0 |
| LET16 | Great Letaba | 0.5045 | 0.3653 | 0 |
| LET18 | Broederstroom | 0.4834 | 0.4151 | 0 |
| Shingwedzi catchment |  |  |  |  |
| SHI1 | Shingwedzi | 0.3576 | 0.345 | 0 |

The cross-sectional views of the EWR sites per river, stage discharge relationships developed from the modellingand the detailed output tables are available electronically on the CD.

The confidence rating in the hydraulic modelling results for the EWR sites ranges from 0=none to 5=high and is indicated in Table 5.

Table 5: Confidence in the hydraulic modelled results

| EWR site | River | Limits of measured discharge range (m3/s) | Confidence rating for discharge range |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  | Q measured | Q< Q measured | Q> Q measured |  |
| Olifants catchment |  |  |  |  |  |
| Olifants_S2 | Lower Wilge | 0.125 | 2 | 1 | Boulders will affect channel roughness during high flows |
| Olifants_S3 | Upper Wilge | 0.004 | 3 | 1 | Downstream bend in the river, bank vegetation and instream boulders will influence results during high flows |
| Olifants_S5 | Olifants | 0.163 | 2 | 1 | Side channel will activate during high flows |
| Olifants_S6 | Klein Olifants | 0.391 | 2 | 1 | In-channel island will develop under high flow conditions |
| Olifants_S7 | Olifants | 2.200 | 2 | 1 | Side channel will activate during high flows as well as boulder bed which will reduce the confidence of the modelling under high flow conditions |
| Olifants_S8 | Kranspoortspruit | 0.012 | 2 | 1 | Upstream bridge will influence the site under high flows |
| Olifants_S9 | Selons | 0.038 | 3 | 2 | Good site characteristics for modelling |
| Olifants_S10 | Olifants | 2.343 | 2 | 1 | Very large system which is difficult to model under high flows. The site is located just upstream of a change in gradient which may influence the hydraulics under varied flow conditions |
| Olifants_S11 | Lower Spekboom | 0.028 | 2 | 1 | In-channel boulders, islands and bank vegetation will influence hydraulics under high flow conditions |
| Olifants_S12 | Upper Blyde | 2.039 | 3 | 1 | Downstream bridge and bend in the river will reduce the confidence in the modelling for high flows. |
| Olifants_S13 | Olifants | 4.051 | 2 | 1 | The site has a series of sand banks and channels that will alter the flow characteristics |
| Olifants_S14 | Lower Blyde | 0.653 | 3 | 1 | A side channel will activate under high flows and this will reduce the confidence in the results under these flow conditions |
| Olifants_S16 | Olifants | 2.964 | 2 | 1 | The wide nature of the site and upstream islands will reduce the confidence in the modelling under high flow conditions. |
| Letaba catchment |  |  |  |  |  |
| LET2 | Letaba | 0.530 | 3 | 1 | The sandy and wide nature of the site will alter under high flows |
| LET14 | Letsitele | 0.668 | 2 | 1 | Upstream weir and variable roughness may alter hydraulics under high flow conditions |
| LET16 | Great Letaba | 0.747 | 3 | 2 | Good site characteristics for hydraulic modelling |
| LET18 | Broeder- stroom | 0.192 | 3 | 2 | Good site characteristics for hydraulic modelling |
| Shingwedzi catchment |  |  |  |  |  |
| SHI1 | Shingwedzi | 0.006 | 2 | 1 | The site is located downstream of a geological control that will influence the hydraulics under high flow conditions. |

## Hydrological data

The natural hydrology for the three catchments were obtained from the following studies, namely:

- Olifants catchment – Development of an Integrated Water Resource Management Plan (IWRMP) for the Upper andMiddle Olifants Catchment. Report No. P WMA 04/000/00/7007. DWA, Directorate National WaterResource Planning. July 2009: Hydrology supporting report.

- Letaba and Shingwedzi catchments – Development of a Reconciliation Strategy for the Luvuvhu and Letaba Water Supply System. Report No. P WMA 02/B810/00/1412/5. DWA, Directorate National Water Resource Planning. March 2014: Hydrology supporting report.

The natural flow time series obtained from these studies were used and adjusted by catchment area to obtain the natural flows at the EWR sites. The natural Mean Annual Runoff (MAR) per EWR site is shown inTable 6. The final natural time series per EWR site is provided electronically on the CD.

Table 6: Natural MAR per EWR site in the Olifants, Letaba and Shingwedzi catchments

| EWR site | Quaternary catchment | River | Natural MAR (106m3) |
| --- | --- | --- | --- |
| Olifants catchment |  |  |  |
| Olifants_S1 | B31C | Elands | 31.075 |
| Olifants_S2 | B20J | Lower Wilge | 175.59 |
| Olifants_S3 | B20F | Upper Wilge | 44.755 |
| Olifants_S5 | B11J | Olifants | 184.54 |
| Olifants_S6 | B12D | Klein Olifants | 66.649 |
| Olifants_S7 | B32A | Olifants | 500.63 |
| Olifants_S8 | B32A | Kranspoortspruit | 4.712 |
| Olifants_S9 | B32C | Selons | 33.109 |
| Olifants_S10 | B71D | Olifants | 813.18 |
| Olifants_S11 | B42H | Lower Spekboom | 148.19 |
| Olifants_S12 | B60B | Upper Blyde | 164.45 |
| Olifants_S13 | B71G | Olifants | 1321.9 |
| Olifants_S14 | B60J | Lower Blyde | 363.27 |
| Olifants_S15 | B72D | Olifants | 1762.1 |
| Olifants_S16 | B73H | Olifants | 1918.3 |
| Olifants_EWR5 | B32D | Olifants | 571.13 |
| Olifants_EWR6 | B31F | Elands | 60.320 |
| Olifants_EWR7 | B51G | Olifants | 726.64 |
| Olifants_EWR9 | B41H | Steelpoort | 137.50 |
| Letaba catchment |  |  |  |
| LET2 | B83D | Letaba | 646.28 |
| LET14 | B81D | Letsitele | 116.55 |
| LET16 | B81B | Great Letaba | 99.85 |
| LET18 | B81A | Broederstroom | 6.683 |
| Shingwedzi catchment |  |  |  |
| SHI1 | B90H | Shingwedzi | 86.424 |

## Quantification of EWRs

The results of the field assessments of the various habitat and biotic components to obtain the EcoStatus and the RECwere completed during the Ecoclassification component (step 3) of the study.

The quantification of the EWRs usedthe Desktop Reserve Model (DRM) (SPATSIM, version 2.12) to calculate the Ecological Water Requirements (quantity) for the REC at the EWR sites.

These EWR flow data were converted to hydraulic conditions at the rapid III EWR sites (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model and evaluated by the ecologists. Where the modelled requirements were not adequate to provide the envisaged protection, the DRM was adjusted accordingly.

The following approaches were used:

- Verification of the drought and base flows (maintenance flows) using the DRM and hydraulic cross-sections;

- Specification of freshets and annual floods at the new rapid sites;

- Assessment of the freshets and annual floods specified during the previous preliminary Reserve studies, taking the release capacities of dams into consideration. These freshets were adjusted where required when higher than the release capacities of the dams;

- Assessment of the equal drought and base flows as specified at specific EWR sites for the Olifants catchment during the 2001 comprehensive study. Adjustments were made to these flow requirements, using the hydraulic cross-sections and habitat availability during drought periods.

These EWR results werethen used to produce the final Ecological Reserve quantity results in the format of an assurance table or EWR rule curves.  These curves specify the frequency of occurrence relationships of the defined maintenance and drought flow requirements for each month of the year.  The tables thus specify the % of time that defined flows should equal or exceed the flow regime required to satisfy the ecological Reserve.

The final EWR results (summary tables, rule tables and long-term requirements) per EWR site is provided electronically on the CD.

## Water Quality

Water quality is being addressed in this study to the extent of reviewing, updating and refining the water quality ecological specifications at the EWR sites and at identified priority areas in the system. Water quality deterioration is occurring in many areas within the catchment. This study will define ecological specifications for water quality where required. The specifications will be defined for water quality as it relates to sustainability and maintenance of the ecological condition and to ensure protection.

## APPROACH

The approach to address the water quality component is as follows:

- Identification of priority sites and areas were water quality is a problem through the gap analysis step (refer to Table 1);

- Review of existing water quality data related to the EWR sites and additional sites/catchments identified;

- Information collected during the field surveys;

- Results from the Eco-classification process (Present Ecological State (PES), Ecological Importance(EI), Ecological Sensitivity (ES) and Recommended Ecological Category (REC);

- Review of results of previous Reserve studies

- Review of resource quality objectives that have been set

- Evaluation of the water quality at specific selected sites where quality was identified as an issue

- Identification of strategic sub-catchments ecological water quality requirements will be specified

- Alignment of RQO requirements at strategic sites and priority sites (EWR and other sites);

- Defining the ecological specifications as identified in the catchment.

Table 7provides a summary of the Resource Quality Objectives for water quality that have been set for rivers in the Olifants River System.  The draft RQOs for the Letaba System have been published for comment and are still to be gazetted.

Table 7: Summary of Resource Quality Objectives for Water Quality (Rivers) for the Olifants River system

| Variable | Units | Bound | Olifants B11 G; B11J (upper portion) | Olifants B11J | Olifants B11L | Klipspruit B11K/L | Wilge B20J | Klein Olifants B12E | Elands (outlet of quat, confluence with Olifants) 31J | Olifants B32C Bottom of quat, outlet) | Olifants 32H (outlet of quat). Included Moses and Mametse (32G and 32H) | Steelpoort confluence with Olifants B41K | Spekboom B42H | Ohrigstad and tributaries (B60E, B60F, B60H) | Ga-Selati (B72K) outlet and EWR site 14b | Olifants EWR site13 B72D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chloride (Cl) | mg/l | Upper |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDS | mg/l | Upper |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EC | mS/m | Upper | 111 |  | 55 | 111 |  | 85 |  | 111 |  |  |  |  | 111 |  |
| SO4 | mg/l | Upper | 500 |  | 80 | 500 | 200 | 200 |  | 500 |  |  |  |  | 500 |  |
| pH | units | Upper |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Phosphate | mg/l P | Upper | 0.125 | 0.125 | 0.015 | 0.125 |  |  |  |  |  | 0.125 |  | 0.125 |  |  |
| Nitrate-Nitrite | mg/l N | Upper | 4 | 4 | 0.7 |  |  |  |  |  |  |  |  | 4 |  |  |
| TIN | mg/l |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ammonia | mg/l N | Upper | 0.1 | 0.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Chl-a phytoplankton | ug/l | Upper |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Akalinity | mg/l (CaCO3) | Upper | 60 |  |  | 60 |  |  |  | 60 |  |  |  |  | 60 |  |
| Turbidity | NTU | Upper | 10 |  |  |  |  |  |  |  |  |  |  |  | 10 |  |
| Dissolved oxygen | mg/l | Upper | 6.5 |  |  | 6.5 |  |  |  | 6.5 |  |  |  |  | 6.5 |  |
| Temperature |  | Upper |  |  |  | ≤abs (dev from ambient) 4.0 |  |  |  | ≤abs (dev from ambient) 4.0 |  |  |  |  | ≤abs (dev from ambient) 4.0 |  |
| Suspended Solids | mg/l | Upper |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 25 |
| F | mg/l | Upper | 3 |  |  | 3 | 2.5 |  |  | 3 |  | 2 | 3 |  | 2.5 |  |
| Al | mg/l | Upper | 0.15 |  |  | 0.15 | 0.105 |  |  | 0.15 |  | 0.063 | 0.15 |  | 0.105 |  |
| As | mg/l | Upper | 0.13 |  |  | 0.13 | 0.095 |  |  | 0.13 |  | 0.058 | 0.13 |  | 0.095 |  |
| Cd(hard) | ug/l | Upper | 5 |  |  | 5 | 3 |  |  | 5 |  | 1.6 | 5 |  | 3 |  |
| Cr (VI) | ug/l | Upper | 200 |  |  | 200 | 121 |  |  | 200 |  | 68 | 200 |  | 121 |  |
| Cu hard | ug/l | Upper | 8 |  |  | 8 | 6 |  |  | 8 |  | 4.9 | 8 |  | 6 |  |
| Hg | ug/l | Upper | 1.7 |  |  | 1.7 | 0.97 |  |  | 1.7 |  | 0.53 | 1.7 |  | 0.97 |  |
| Mn | mg/l | Upper | 1.3 |  |  | 1.3 | 0.99 |  |  | 1.3 |  | 0.68 | 1.3 |  | 0.99 |  |
| Pb hard | ug/l | Upper | 13 |  |  | 13 | 9.5 |  |  | 13 |  | 5.8 | 13 |  | 9.5 |  |
| Se | mg/l | Upper | 0.03 |  |  | 0.03 | 0.022 |  |  | 0.03 |  | 0.013 | 0.03 |  | 0.022 |  |
| Zn | ug/l | Upper | 36 |  |  | 36 | 25.2 |  |  | 36 |  | 14.4 | 36 |  | 25.2 |  |
| Chlorine | ug/l | Upper | 5.0 free Cl |  |  | 5.0 free Cl | 3.1 free Cl |  |  | 5 free Cl |  | 1.8 free Cl | 5 free Cl |  | 3.1 free Cl |  |
| Endosulfan | ug/l | Upper | 0.2 |  |  | 0.2 | 0.13 |  |  | 0.2 |  | 0.08 | 0.2 |  | 0.13 |  |
| Atrazine | ug/l | Upper | 100 |  |  | 100 | 78.5 |  |  | 100 |  | 48.8 | 100 |  | 78.5 |  |
| Pathogens | counts/100ml E. coli | Upper |  |  |  |  | 130 |  | 130 |  | 130 |  |  |  |  |  |

# RESULTS (QUANTITY)

The results of the ecological Reserve determination of the various rivers in the Olifants, Letaba and Shingwedzi catchments at the selected EWR sites are presented in this section. These include the new rapid sites, re-surveying of existing EWR sites and the re-assessment of the EWRs using existing hydraulics and biological data.

## Olifants_S1: Upper Elands River in B31C

This site was assessed on a rapid level 2 as no hydraulic cross-section was available. The site is situated just downstream of a low water bride and formed multiple channels that would provide very low confidence in the hydraulic results (Figure 3).

Figure 3: View of the EWR site on the Upper Elands River

The EWRfor the upper Elands River in quaternary catchment B31C were determined for a REC of C.

As no hydraulic cross-section was available, the DRM results were compared to the discharge of 0.009 m3/s measuredon 7 October 2015. The drought and maintenance low flows recommended by the DRM for October are 0.038 m3/s and 0.080 m3/s. These flows are much higher than the observed discharge, thus the DRM recommendations were accepted. No large dams are situated in the upper catchment and the freshets as recommended by the DRM were accepted.

Table 8summarises thefinal EWR for the Upper Elands River in quaternary catchment B31C.

Table 8: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B31C |
| --- | --- |
| River | Upper Elands |
| EWR Site Co-ordinates | S25.3031, E28.4631 |
| Recommended Ecological Category | C |
| VMAR at EWR site | 31.075 |
| Total EWR | 6.485 (20.87 %MAR) |
| Maintenance Low flows | 3.296 (10.61 %MAR) |
| Drought Low flows | 1.533 ( 4.93 %MAR) |
| Maintenance High flows | 3.189 (10.26 %MAR) |
| Overall confidence | Low |

## Olifants_S2: Lower Wilge River in B20J

This site was assessed on a rapid level 3 and is situated in the vicinity of the existing Olifants_EWR4 site from the 2001 comprehensive Reserve study. The habitat during the survey was dominatedby stones out of current and small cobbles due to the very low flows (Figure 4).

Figure 4: View of the EWR site on the Lower Wilge River

The EWRfor the lower Wilge River in quaternary catchment B20J were determined for a REC of a B.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September andFebruary. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 8 October 2015. The discharge at the EWR site during the site visit was 0.125m3/s(Figure 5) and was used as reference to adjust the recommended EWRs.

Figure 5: Water levels on cross-section of the EWR site for the Lower Wilge River in B20J

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements.The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended by the DRM are 0.799 m3/s and 0.245 m3/s respectively, compared to the measured discharge of 0.125 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows for a B category were accepted for the Lower Wilge River in quaternary catchment B20J.

The freshets and annual flood as specified for this site during the 2001 comprehensive study were assessed and some adjustments were made to derive the final results. These included changing from specifying freshets for drought years and freshets for maintenance years specifying to only one set of freshets for ease of implementation. The initial and final freshets and floods for the lower Wilge River is provided inTable 9.

Table 9: Freshets and floods for the 2001 study and final requirements for implementation

| Months | 2001 Comprehensive study, Olifants_EWR4 |  |  |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Maintenance freshets |  | Maintenance floods |  | Drought freshets |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days | m3/s | days |
| October | 6 | 2 |  |  |  |  | 5 | 2 |
| November | 5 | 2 | 10 | 3 | 5 | 2 | 14 | 3 |
| December | 14 | 3 |  |  | 8 | 2 | 22 | 3 |
| January | 34 | 3 |  |  |  |  | 34 | 3 |
| February | 45 | 4 |  |  | 8 | 2 | 45 | 4 |
| March | 34 | 4 | 14 | 3 |  |  | 34 | 3 |
| April | 5 | 2 |  |  |  |  | 5 | 2 |
| September |  |  |  |  |  |  | 2 | 2 |

Table 10gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 11.

Table 10: EWR results for specific months for the Lower Wilge River in B20J (REC = B)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 0.690 | 0.43 | 0.19 | 0.17 |
| High flows | February | 1.861 | 0.57 | 0.29 | 0.26 |
| Datum | October | 0.799 | 0.45 | 0.21 | 0.18 |
| Drought flows | October | 0.245 | 0.32 | 0.14 | 0.12 |
| Measured discharge at site visit (8 October 2015) |  | 0.125 | 0.267 | 0.11 | 0.091 |

Table 11: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B20J |
| --- | --- |
| River | Lower Wilge |
| EWR Site Co-ordinates | S25.6196, E28.9991 |
| Recommended Ecological Category | B |
| VMAR at EWR site | 175.58 |
| Total EWR | 63.698 (36.28%MAR) |
| Maintenance Low flows | 37.907 (21.59%MAR) |
| Drought Low flows | 11.029 (6.28%MAR) |
| Maintenance High flows | 25.790 (14.69 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_S3: Upper Wilge River in B20F

This site was assessed on a rapid level 3 and is situated just downstream of the R104 from Bronkhorsstspruit. Limited habitats were available due to the very low flows during the surveys (Figure 6).

Figure 6: View of the EWR site on the Upper Wilge River

The EWRfor the upper Wilge River in quaternary catchment B20F were determined for a REC of C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 8 October 2015. The discharge at the EWR site during the site visit was 0.1004 m3/s(Figure 7) and was used as reference to adjust the recommended EWRs.

Figure 7: Water levels on cross-section of the EWR site for the Upper Wilge River in B20F

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended by the DRM for October are 0.059 m3/s and 0.124 m3/s respectively, compared to the measured discharge of 0.004 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was more than adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended maintenance flows for a C category were adjusted as follows:

Maintenance low flows for October: Adjusted from 0.124 m3/s to 0.085 m3/s

No changes were made to the recommended drought flows. The freshets and annual flood requirements for the site were adjusted and are specified inTable 12.

Table 12: Freshets and floods for the Upper Wilge EWR site in B20F

| Months | Final freshets/ floods |  |
| --- | --- | --- |
|  | m3/s | days |
| October | 1.2 | 3 |
| November | 1.2 | 3 |
| December | 4.5 | 3 |
| January | 4.5 | 3 |
| February | 5.0 | 3 |
| March | 4.5 | 3 |
| April | 1.2 | 3 |

Table 13gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 14.

Table 13: EWR results for specific months for the Upper Wilge River in B20F (REC = C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 0.081 | 0.16 | 0.1 | 0.15 |
| High flows | February | 0.150 | 0.21 | 0.14 | 0.19 |
| Datum | October | 0.085 | 0.165 | 0.1 | 0.15 |
| Drought flows | October | 0.059 | 0.14 | 0.09 | 0.13 |
| Measured discharge at site visit (8 October 2015) |  | 0.004 | 0.048 | 0.04 | 0.04 |

Table 14: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B20F |
| --- | --- |
| River | Upper Wilge |
| EWR Site Co-ordinates | S25.8439, E28.8719 |
| Recommended Ecological Category | C |
| VMAR at EWR site | 44.755 |
| Total EWR | 6.763 (15.11 %MAR) |
| Maintenance Low flows | 3.326 ( 7.43 %MAR) |
| Drought Low flows | 2.151 ( 4.80 %MAR) |
| Maintenance High flows | 3.437 ( 7.68 %MAR) |
| Overall confidence | Low |

## Olifants_S5: Olifants River in B11J

This site was assessed on a rapid level 3 and is situated just upstream of the existing Olifants_EWR1 site from the 2001 comprehensive Reserve study. This site was selected as the initial site is inundated by the construction of a weir. The habitat during the survey was dominatedby stones out of current, large and small boulders and bedrock sections due to the low flows (Figure 8).

Figure 8: View of the EWR site on the Olifants River

The EWRfor the Olifants River in quaternary catchment B11J were determined for a REC of C/D.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor August and February. August is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 9 October 2015. The discharge at the EWR site during the site visit was 0.163m3/s(Figure 9) and was used as reference to adjust the recommended EWRs.

Figure 9: Water levels on cross-section of the EWR site for the Olifants River in B11J

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended for October by the DRM are 0.161 m3/s and 0.258 m3/s respectively, compared to the measured discharge of 0.163 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October (maintenance flows) and August (drought flows) were not adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows for a C/D category were adjusted as follows:

Maintenance low flows for October: Adjusted from 0.258 m3/s to 0.380 m3/s

Drought flows for August: Adjusted from 0.114 m3/s to 0.151 m3/s

The freshets and annual flood as specified for this site during the 2001 comprehensive study were assessed and some adjustments were made to derive the final results. These included changing from specifying freshets for drought years and freshets for maintenance years specifying to only one set of freshets for ease of implementation. The initial and final freshets and floods for the Olifants River in B11J is provided inTable 15.

.

Table 15: Freshets and floods for the 2001 study and final requirements for implementation

| Months | 2001 Comprehensive study, Olifants_EWR1 |  |  |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Maintenance freshets |  | Maintenance floods |  | Drought freshets |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days | m3/s | days |
| October | 3 | 1 |  |  | 2 | 1 | 3 | 2 |
| November | 3 | 2 | 5 | 2 | 4 | 1 | 10 | 2 |
| December | 10 | 3 | 30 | 3 | 4 | 1 | 10 | 2 |
| January | 10 | 3 | 10 | 3 | 2.5 | 3 | 20 | 4 |
| February | 100 | 4 |  |  | 4 | 1 | 20 | 4 |
| March | 10 | 3 |  |  |  |  | 10 | 2 |
| April | 3 | 1 |  |  | 2 | 1 | 3 | 2 |

Table 16gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 17.

Table 16: EWR results for specific months for the Olifants River in B11J (REC = C/D)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | August | 0.258 | 0.41 | 0.17 | 0.17 |
| High flows | February | 1.437 | 0.67 | 0.32 | 0.32 |
| Datum | October | 0.380 | 0.46 | 0.2 | 0.2 |
| Drought flows | August | 0.151 | 0.35 | 0.12 | 0.15 |
| Measured discharge at site visit (9 October 2015) |  | 0.163 | 0.363 | 0.13 | 0.16 |

Table 17: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B11J |
| --- | --- |
| River | Olifants |
| EWR Site Co-ordinates | S25.7592, E29.3096 |
| Recommended Ecological Category | C/D |
| VMAR at EWR site | 184.53 |
| Total EWR | 33.988 (18.42 %MAR) |
| Maintenance Low flows | 22.375 (12.13 %MAR) |
| Drought Low flows | 8.291 ( 4.49 %MAR) |
| Maintenance High flows | 11.612 ( 6.29 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_S6: Klein Olifants River in B12D

This site was assessed on a rapid level 3 and is situated just downstream the N11 bridge in Middelburg. The habitat during the survey included stones in and out of current, large and small cobblesand bedrock sections (Figure 10). The flow was moderate during the surveys.

Figure 10: View of the EWR site on the Klein Olifants River

The EWRfor the Klein Olifants River in quaternary catchment B12D were determined for a REC of C/D.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 9 October 2015. The discharge at the EWR site during the site visit was 0.391m3/s(Figure 11) and was used as reference to adjust the recommended EWRs.

Figure 11: Water levels on cross-section of the EWR site for the Klein Olifants River in B12D

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended by the DRM for October are 0.060 m3/s and 0.099 m3/s respectively, compared to the measured discharge of 0.391 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was not adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows for a C/D category were adjusted as follows:

Maintenance low flows for October: Adjusted from 0.099 m3/s to 0.210 m3/s

Drought flows for October: Adjusted from 0.060 m3/s to 0.086 m3/s

Although the 2001 EWR site for the Klein Olifants River was downstream in the gorge area in B12E, the freshets and annual flood specified for this site were used as a guide for the specification of the freshets at the new site. These are provided inTable 18.

Table 18: Freshets and floods for the Klein Olifants EWR site in B12D

| Months | Final for implementation |  |
| --- | --- | --- |
|  | Final freshets/ floods |  |
|  | m3/s | days |
| October | 1.2 | 2 |
| November | 8 | 2 |
| December | 10 | 3 |
| January | 15 | 3 |
| February | 15 | 3 |
| March | 6 | 2 |
| April | 2.5 | 2 |

Table 19gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 20.

Table 19: EWR results for specific months for the Klein Olifants River in B12D (REC = C/D)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 0.179 | 0.23 | 0.08 | 0.25 |
| High flows | February | 0.516 | 0.32 | 0.15 | 0.34 |
| Datum | October | 0.210 | 0.24 | 0.09 | 0.26 |
| Drought flows | October | 0.086 | 0.19 | 0.08 | 0.22 |
| Measured discharge at site visit (9 October 2015) |  | 0.391 | 0.29 | 0.13 | 0.31 |

Table 20: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B12D |
| --- | --- |
| River | Klein Olifants |
| EWR Site Co-ordinates | S25.7489, E29.4587 |
| Recommended Ecological Category | C/D |
| VMAR at EWR site | 66.649 |
| Total EWR | 18.312 (27.47 %MAR) |
| Maintenance Low flows | 9.950 (14.93 %MAR) |
| Drought Low flows | 3.673 ( 5.51 %MAR) |
| Maintenance High flows | 8.362 (12.55 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_S7: Olifants River in B32A

This site was assessed on a rapid level 3 and is situated in the vicinity of the existing Olifants_EWR2 site from the 2001 comprehensive Reserve study. The habitat during the survey was dominatedby stones out of current, large cobblesand limited marginal vegetation due to the low flows (Figure 12).

Figure 12: View of the EWR site on the Olifants River in B32A

The EWRfor the Olifants River in quaternary catchment B32A were determined for a REC of B/C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 10 October 2015. The discharge at the EWR site during the site visit was 2.2m3/s(Figure 13) and was used as reference to adjust the recommended EWRs.

Figure 13: Water levels on cross-section of the EWR site for the Olifants River in B32A

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended by the DRM for October are 0.624 m3/s and 1.649 m3/s respectively, compared to the measured discharge of 2.20 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was not adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows for a B/C category were adjusted as follows:

Maintenance low flows for October: Adjusted from 1.649 m3/s to 2.201 m3/s

No adjustments were made to the drought flows.

The freshets and annual flood as specified for this site during the 2001 comprehensive study were assessed and some adjustments were made to derive the final results. These included changing from specifying freshets for drought years and freshets for maintenance years to only one set of freshets for ease of implementation. The initial and final freshets and floods for the Olifants River in B32A is provided inTable 21.

Table 21: Freshets and floods for the 2001 study and final requirements for implementation

| Months | 2001 Comprehensive study, Olifants_EWR2 |  |  |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Maintenance freshets |  | Maintenance floods |  | Drought freshets |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days | m3/s | days |
| October | 5 | 2 |  |  | 3 | 1 | 5 | 2 |
| November | 12 | 3 | 12 | 3 | 6 | 2 | 15 | 3 |
| December | 12 | 3 | 15 | 3 | 6 | 2 | 15 | 3 |
| January | 35 | 4 | 12 | 3 | 20 | 3 | 35 | 4 |
| February | 120 | 5 | 140 | 5 | 15 | 4 | 90 | 5 |
| March | 15 | 3 |  |  |  |  | 1 | 3 |
| April | 5 | 2 |  |  |  |  | 5 | 2 |
| September |  |  |  |  |  |  | 4 | 2 |

Table 22gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 23.

Table 22: EWR results for specific months for the Olifants River in B32A (REC = B/C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 1.717 | 0.43 | 0.25 | 0.55 |
| High flows | February | 6.299 | 0.65 | 0.35 | 0.96 |
| Datum | October | 2.201 | 0.46 | 0.27 | 0.62 |
| Drought flows | October | 0.624 | 0.31 | 0.15 | 0.37 |
| Measured discharge at site visit (10 October 2015) |  | 2.200 | 0.46 | 0.27 | 0.62 |

Table 23: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B32A |
| --- | --- |
| River | Olifants |
| EWR Site Co-ordinates | S25.4963, E29.2546 |
| Recommended Ecological Category | B/C |
| VMAR at EWR site | 500.63 |
| Total EWR | 149.364 (29.83 %MAR) |
| Maintenance Low flows | 115.720 (23.11 %MAR) |
| Drought Low flows | 30.564 ( 6.10 %MAR) |
| Maintenance High flows | 33.644 ( 6.72 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_S8: Kranspoortspruit in B32A

This site was assessed on a rapid level 3 and is situated just downstream of a low water bridge, upstream of Loskop Dam. This site was also assessed on a rapid level 3 during the WRCS study in 2011. The habitat during the survey was dominatedby bedrock and boulder/cobbles with some sand and gravel with very low flows (Figure 14).

Figure 14: View of the EWR site on the Kranspoortspruit

The EWRfor the Kranspoortspruit in quaternary catchment B32A were determined for a REC of B.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor August and February. August is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 11 October 2015. The discharge at the EWR site during the site visit was 0.012 m3/s(Figure 15) and was used as reference to adjust the recommended EWRs.

Figure 15: Water levels on cross-section of the EWR site for the Kranspoortspruit in B32A

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended by the DRM for October are 0.001 m3/s and 0.019 m3/s respectively, compared to the measured discharge of 0.012 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was not adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows for a B category were adjusted as follows:

Maintenance low flows for October: Adjusted from 0.019 m3/s to 0.030 m3/s

No adjustments were made to the drought flows, freshets and floods specified by the DRM.

Table 24gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 25

Table 24: EWR results for specific months for the Kranspoortspruit in B32A (REC = B)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | August | 0.020 | 0.16 | 0.08 | 0.02 |
| High flows | February | 0.073 | 0.24 | 0.15 | 0.03 |
| Datum | October | 0.030 | 0.19 | 0.1 | 0.02 |
| Measured discharge at site visit (11 October 2015) |  | 0.012 | 0.14 | 0.06 | 0.015 |

Table 25: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B32A |
| --- | --- |
| River | Kranspoortspruit |
| EWR Site Co-ordinates | S25.4377, E29.4756 |
| Recommended Ecological Category | B |
| VMAR at EWR site | 4.712 |
| Total EWR | 1.940 (41.16 %MAR) |
| Maintenance Low flows | 1.307 (27.75 %MAR) |
| Drought Low flows | 0.084 ( 1.78 %MAR) |
| Maintenance High flows | 0.632 (13.42 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_S9: Selons River in B32C

This site was assessed on a rapid level 3 and is situated after the confluence with the Kruis River. The habitats dominating during the surveys were stones out of current and small cobbles due to the very low flows. Some cobbles and boulders were present (Figure 16).

The EWRfor the Selons River in quaternary catchment B32C were determined for a REC of C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 11 October 2015. The discharge at the EWR site during the site visit was 0.035 m3/s(Figure 17) and was used as reference to adjust the recommended EWRs.

Figure 16: View of the EWR site on the Selons River

Figure 17: Water levels on cross-section of the EWR site for the Selons River in B32C

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended for October by the DRM are 0.050 m3/s and 0.098 m3/s respectively, compared to the measured discharge of 0.035 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for the maintenance flows for October was adequate to provide the necessary velocities and habitats. However, the drought flows recommended for October (0.050 m3/s) were inadequate and it was adjusted as follows:

Drought flows for October: Adjusted from 0.050 m3/s to 0.057 m3/s

Drought flows for September: Adjusted from 0.048 m3/s to 0.055 m3/s

No adjustments were made to the maintenance flows, freshets and floods specified by the DRM.

Table 26gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 27.

Table 26: EWR results for specific months for the Selons River in B32C (REC = C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 0.094 | 0.18 | 0.09 | 0.21 |
| High flows | February | 0.193 | 0.24 | 0.12 | 0.27 |
| Datum | October | 0.098 | 0.19 | 0.09 | 0.22 |
| Drought flows | October | 0.050 | 0.15 | 0.07 | 0.18 |
| Measured discharge at site visit (11 October 2015) |  | 0.035 | 0.133 | 0.06 | 0.173 |

Table 27: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B32C |
| --- | --- |
| River | Selons |
| EWR Site Co-ordinates | S25.3799, E29.4356 |
| Recommended Ecological Category | C |
| VMAR at EWR site | 33.109 |
| Total EWR | 7.237 (21.86 %MAR) |
| Maintenance Low flows | 4.130 (12.47 %MAR) |
| Drought Low flows | 2.356 ( 7.11 %MAR) |
| Maintenance High flows | 3.107 ( 9.39 %MAR) |
| Overall confidence | Low |

## Olifants_S10: Olifants River in B71D

This site was assessed on a rapid level 3 and is situated just downstream of the confluence with the Mohlapitse River in the vicinity of the existing Olifants_EWR8 site from the 2001 comprehensive Reserve study. Habitats available during the survey were stones in and out of current and bedrock. Limited boulders, sand and marginal vegetation were available due to the low flows (Figure 18).

Figure 18: View of the EWR site on the Olifants in B71D

The EWRfor the Olifants River in quaternary catchment B71D were determined for a REC of C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 12 October 2015. The discharge at the EWR site during the site visit was 2.343m3/s(Figure 19)and was used as reference to adjust the recommended EWRs.

Figure 19: Water levels on cross-section of the EWR site for the Olifants River in B71D

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended by the DRM are 1.002 m3/s and 1.929 m3/s respectively, compared to the measured discharge of 2.343 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was not adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows for a C category were adjusted as follows:

Maintenance low flows for October: Adjusted from 1.929 m3/s to 2.335 m3/s

No adjustments were made for the drought flows.

The freshets and annual flood as specified for this site during the 2001 comprehensive study were assessed and some adjustments were made to derive the final results. These included changing from specifying freshets for drought years and freshets for maintenance years separately, to only one set of freshets for ease of implementation. The initial and final freshets and floods for the Olifants River in B71D is provided inTable 28.

.

Table 28: Freshets and floods for the 2001 study and final requirements for implementation

| Months | 2001 Comprehensive study, Olifants_EWR8 |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maintenance freshets |  | Drought freshets |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days |
| October | 10 | 2 | 5 | 2 | 5 | 2 |
| November | 20 | 3 | 10 | 2 | 10 | 2 |
| December | 35 | 4 | 20 | 3 | 20 | 4 |
| January | 50 | 5 | 35 | 4 | 35 | 5 |
| February | 350 | 6 | 60 | 5 | 100 | 6 |
| March | 35 | 4 | 20 | 3 | 35 | 5 |
| April | 10 | 2 |  |  | 5 | 2 |

Table 29gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 30.

Table 29: EWR results for specific months for the Olifants River in B71D (REC = C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 1.865 | 0.42 | 0.22 | 0.34 |
| High flows | February | 7.096 | 0.64 | 0.42 | 0.62 |
| Datum | October | 2.335 | 0.45 | 0.25 | 0.37 |
| Drought flows | September | 0.819 | 0.32 | 0.14 | 0.25 |
| Measured discharge at site visit (12 October 2015) |  | 2.343 | 0.451 | 0.25 | 0.37 |

Table 30: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B71D |
| --- | --- |
| River | Olifants |
| EWR Site Co-ordinates | S24.2399, E30.0825 |
| Recommended Ecological Category | C |
| VMAR at EWR site | 813.17 |
| Total EWR | 169.747 (20.87 %MAR) |
| Maintenance Low flows | 126.469 (15.55 %MAR) |
| Drought Low flows | 50.939 ( 6.26 %MAR) |
| Maintenance High flows | 43.278 ( 5.32 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_S11: Lower Spekboom River in B42H

This site was assessed on a rapid level 3 and is situated upstream of Burgersfort and the confluence with the Steelpoort River. Limited habitats were available during the survey due to the very low flows. Available habitats were dominatedby stones out of current and small cobbles (Figure 20).

Figure 20: View of the EWR site on the Lower Spekboom River

The EWRfor the lower Spekboom River in quaternary catchment B42H were determined for a REC of B/C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 12 October 2015. The discharge at the EWR site during the site visit was 0.028m3/s(Figure 21) and was used as reference to adjust the recommended EWRs.

Figure 21: Water levels on cross-section of the EWR site for the Lower Spekboom River in B42H

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed for October by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows for October recommended by the DRM are 0.214 m3/s and 0.571 m3/s respectively, compared to the measured discharge of 0.028 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows for a B/C category were accepted for the Lower Spekboom River in quaternary catchment B42H.

The freshets as specified by the DRM were also accepted.

Table 31gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 32.

Table 31: EWR results for specific months for the Lower Spekboom River in B42H (REC = B/C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 0.550 | 0.38 | 0.23 | 0.18 |
| High flows | February | 1.234 | >0.4 | >0.24 | >0.19 |
| Datum | October | 0.571 | 0.39 | 0.24 | 0.19 |
| Drought flows | October | 0.214 | 0.26 | 0.14 | 0.48 |
| Measured discharge at site visit (12 October 2015) |  | 0.028 | 0.121 | 0.07 | 0.09 |

Table 32: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B42H |
| --- | --- |
| River | Lower Spekboom |
| EWR Site Co-ordinates | S24.6942, E30.3613 |
| Recommended Ecological Category | B/C |
| VMAR at EWR site | 148.19 |
| Total EWR | 45.634 (30.79 %MAR) |
| Maintenance Low flows | 25.803 (17.41 %MAR) |
| Drought Low flows | 9.346 ( 6.31 %MAR) |
| Maintenance High flows | 19.831 (13.38 %MAR) |
| Overall confidence | Low |

## Olifants_S12: Upper Blyde River in B60B

This site was assessed on a rapid level 3 and is situated upstream of the Treur River confluence. The habitats available during the survey werestones in and out of current, cobbles and boulders with snagged trees under the bridge providing additional habitat(Figure 22). The flows were moderate during the survey.

Figure 22: View of the EWR site on the Upper Blyde River

The EWRfor the upper Blyde River in quaternary catchment B60B were determined for a REC of B.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 13 October 2015. The discharge at the EWR site during the site visit was 2.039m3/s(Figure 23) and was used as reference to adjust the recommended EWRs.

Figure 23: Water levels on cross-section of the EWR site for the Upper Blyde River in B60B

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended for October by the DRM are 0.358 m3/s and 1.115 m3/s respectively, compared to the measured discharge of 2.039 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was not adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows were adjusted as follows:

Maintenance low flows for October: Adjusted from 1.115 m3/s to 1.439 m3/s

Drought flows for October: Adjusted from 0.358 m3/s to 0.503 m3/s

The freshets and annual flood as specified for this site by the DRM was adjusted as in Table 33.

Table 33: Freshets and floods requirements for implementation

| Months | Final for implementation |  |
| --- | --- | --- |
|  | Final freshets/ floods |  |
|  | m3/s | days |
| October | 5 | 2 |
| November | 5 | 2 |
| December | 15 | 2 |
| January | 15 | 2 |
| February | 54* | 2 |
| March | 28* | 2 |
| April | 13* | 2 |

* Accepted the DRM freshet requirements

Table 34gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 35.

Table 34: EWR results for specific months for the Upper Blyde River in B60B (REC = B)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 1.501 | 0.37 | 0.13 | 0.67 |
| High flows | February | 2.604 | 0.42 | 0.16 | 0.81 |
| Datum | October | 1.439 | 0.36 | 0.12 | 0.66 |
| Drought flows | October | 0.503 | 0.28 | 0.09 | 0.51 |
| Measured discharge at site visit (13 October 2015) |  | 2.039 | 0.392 | 0.15 | 0.74 |

Table 35: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B60B |
| --- | --- |
| River | Upper Blyde |
| EWR Site Co-ordinates | S24.7344, E30.7783 |
| Recommended Ecological Category | B |
| VMAR at EWR site | 164.45 |
| Total EWR | 75.777 (46.08 %MAR) |
| Maintenance Low flows | 59.129 (35.96 %MAR) |
| Drought Low flows | 20.003 (12.16 %MAR) |
| Maintenance High flows | 16.648 (10.12 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_S13: Olifants River in B71J

This site was assessed on a rapid level 3 and is situated upstream of the Blyde River confluence in the vicinity of the existing Olifants_EWR11 site from the 2001 comprehensive Reserve study. The habitats available during the survey werestones in and out of current, cobbles and limited marginal vegetation due to the low flows (Figure 24).

Figure 24: View of the EWR site on the Olifants River in B71J

The EWRfor the OlifantsRiver in quaternary catchment B71J were determined for a REC of C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 14 October 2015. The discharge at the EWR site during the site visit was 4.051m3/s(Figure 25) and was used as reference to adjust the recommended EWRs.

Figure 25: Water levels on cross-section of the EWR site for the Olifants River in B71J

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows for October recommended by the DRM are 1.852 m3/s and 3.449 m3/s respectively, compared to the measured discharge of 0.125 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows for a C category were accepted for the Olifants River in quaternary catchment B71J.

The freshets and annual flood as specified for this site during the 2001 comprehensive study were assessed and some adjustments were made to derive the final results. These included changing from specifying freshets for drought years and freshets for maintenance years respectively to only one set of freshets for ease of implementation. The initial and final freshets and floods for the Olifants River in B71J is provided inTable 36.

.

Table 36: Freshets and floods for the 2001 study and final requirements for implementation

| Months | 2001 Comprehensive study, Olifants_EWR11 |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maintenance freshets |  | Drought freshets |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days |
| October | 9 | 2 | 6 | 2 | 12 | 3 |
| November | 11 | 3 | 6 | 2 | 15 | 3 |
| December | 20 | 4 | 10 | 2 | 20 | 4 |
| January | 35 | 4 | 10 | 2 | 40 | 4 |
| February | 80 | 6 | 30 | 4 | 120 | 6 |
| March | 30 | 4 | 10 | 2 | 30 | 3 |
| April | 13 | 3 |  |  | 10 | 2 |

Table 37gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 38.

Table 37: EWR results for specific months for the Olifants River in B71J (REC = C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 3.011 | 0.5 | 0.28 | 0.46 |
| High flows | February | 10.190 | 0.72 | 0.48 | 0.82 |
| Datum | October | 3.449 | 0.51 | 0.29 | 0.47 |
| Drought flows | September | 1.642 | 0.41 | 0.2 | 0.36 |
| Measured discharge at site visit (14 October 2015) |  | 4.051 | 0.541 | 0.32 | 0.518 |

Table 38: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B71J |
| --- | --- |
| River | Olifants |
| EWR Site Co-ordinates | S24.3076, E30.7857 |
| Recommended Ecological Category | C |
| VMAR at EWR site | 1 321.92 |
| Total EWR | 236.022 (17.85 %MAR) |
| Maintenance Low flows | 187.586 (14.19 %MAR) |
| Drought Low flows | 97.346 ( 7.36 %MAR) |
| Maintenance High flows | 48.436 ( 3.66 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_S14: Lower Blyde River in B60J

This site was assessed on a rapid level 3 and is situated in the vicinity of the existing Olifants_EWR12 site from the 2001 comprehensive Reserve study. The habitat during the survey was dominated by cobbles with snags and root wads (Figure 26). The flows were low during the survey.

Figure 26: View of the EWR site on the Lower Blyde River

The EWRfor the lower Wilge River in quaternary catchment B20J were determined for a REC of B.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 13 October 2015. The discharge at the EWR site during the site visit was 0.653m3/s(Figure 27) and was used as reference to adjust the recommended EWRs.

Figure 27: Water levels on cross-section of the EWR site for the Lower Blyde River in B60J

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended for October by the DRM are 0.670 m3/s and 2.006 m3/s respectively, compared to the measured discharge of 0.653 m3/s.

The consensus reached by the aquatic ecologists was that the maintenance low flows’ velocities and availability of habitats provided by the DRM for October was adequate to provide the necessary velocities and habitats. However, the drought flows recommended for September (0.686 m3/s) was to low and was adjusted as follows:

Drought flows for September: Adjusted from 0.696 m3/s to 0.915 m3/s

The freshets and annual flood as specified for this site during the 2001 comprehensive study were assessed and some adjustments were made to derive the final results. These included changing from specifying freshets for drought years and freshets for maintenance years separately to only one set of freshets for ease of implementation. The initial and final freshets and floods for the lower Wilge River is provided inTable 39.

Table 39: Freshets and floods for the 2001 study and final requirements for implementation

| Months | 2001 Comprehensive study, Olifants_EWR12 |  |  |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Maintenance freshets |  | Maintenance floods |  | Drought freshets |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days | m3/s | days |
| October | 10 | 3 |  |  | 5 | 2 | 2.5 | 3 |
| November | 10 | 3 | 10 | 3 | 10 | 2 | 10 | 3 |
| December | 1 | 3 | 15 | 3 | 10 | 2 | 15 | 3 |
| January | 15 | 3 | 20 | 3 | 10 | 2 | 20 | 3 |
| February | 15 | 3 | 30 | 4 | 20 | 3 | 35 | 3 |
| March | 15 | 3 | 20 | 3 | 15 | 2 | 30 | 3 |
| April | 10 | 3 | 10 | 3 | 5 | 2 | 10 | 3 |

Table 40gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 41.

Table 40: EWR results for specific months for the Lower Blyde River in B60J (REC = B)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 2.086 | 0.43 | 0.25 | 0.57 |
| High flows | February | 4.877 | 0.55 | 0.33 | 0.86 |
| Datum | October | 2.006 | 0.42 | 0.25 | 0.55 |
| Drought flows | September | 0.915 | 0.34 | 0.18 | 0.39 |
| Measured discharge at site visit (13 October 2015) |  | 0.653 | 0.303 | 0.16 | 0.338 |

Table 41: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B60J |
| --- | --- |
| River | Lower Blyde |
| EWR Site Co-ordinates | S24.4075, E30.8274 |
| Recommended Ecological Category | B |
| VMAR at EWR site | 363.27 |
| Total EWR | 119.394 (32.87 %MAR) |
| Maintenance Low flows | 100.343 (27.62 %MAR) |
| Drought Low flows | 40.967 (11.28 %MAR) |
| Maintenance High flows | 19.051 ( 5.24 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_S15: Olifants River in B72D

This site was assessed on a rapid level 2(no hydraulic cross-section was surveyed due to site characteristics) and is situated downstream the Blyde River confluence in the vicinity of the existing Olifants_EWR13 site from the 2001 comprehensive Reserve study. The habitat during the survey was dominatedby sand with some cobbles. The flows were very low during the survey (Figure 28).

Figure 28: View of the EWR site on the Olifants River in B72D

The EWRfor the Olifants River in quaternary catchment B72D were determined for a REC of B/C.

As no hydraulic cross-section was available, the DRM results were compared to the discharge of 1.660 m3/s measuredon 14 October 2015. The drought and maintenance low flows recommended by the DRM for October are 2.730 m3/s and 6.737 m3/s. As the recommended flows are higher that the observed flow, the recommendations were also compared with the 2001 study requirements. The 2001 study assessed the requirements for a B category and specified the following flows for October:

Maintenance flows = 8.0 m3/s

Drought flows = 2.2 m3/s

As this is the last site before the Olifants River enters the Kruger National Park (KNP), and the recommendation for the KNP is a B category, the DRM requirements were adjusted to the 2001 results. Thus, the following changes were made:

Maintenance low flows for October: Adjusted from 6.737 m3/s to 8.029 m3/s

Drought flows for October: Adjusted from 2.730 m3/s to 2.218 m3/s

The freshets and annual flood as specified for this site during the 2001 comprehensive study were assessed and some adjustments were made to derive the final results. These included changing from specifying freshets for drought years and freshets for maintenance years separately to only one set of freshets for ease of implementation. The initial and final freshets and floods for the Olifants River is provided inTable 42.

Table 42: Freshets and floods for the 2001 study and final requirements for implementation

| Months | 2001 Comprehensive study, Olifants_EWR13 |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maintenance freshets |  | Drought freshets |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days |
| October | 15 | 3 | 5 | 2 | 15 | 3 |
| November | 50 | 5 | 10 | 3 | 30 | 3 |
| December | 30 | 4 | 20 | 4 | 50 | 4 |
| January | 60 | 5 | 30 | 4 | 60 | 5 |
| February | 180 | 7 | 60 | 5 | 180 | 7 |
| March | 60 | 5 | 30 | 3 | 60 | 5 |
| April | 20 | 4 | 8 | 2 | 20 | 4 |

Table 43 summarises the final EWR results for the Olifants River in quaternary catchment B72D.

Table 43: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B72D |
| --- | --- |
| River | Olifants |
| EWR Site Co-ordinates | S24.1284, E31.0146 |
| Recommended Ecological Category | B/C |
| VMAR at EWR site | 1 762.1 |
| Total EWR | 527.654 (29.94 %MAR) |
| Maintenance Low flows | 437.971 (24.85 %MAR) |
| Drought Low flows | 114.375 ( 6.49 %MAR) |
| Maintenance High flows | 89.683 ( 5.09 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_S16: Olifants River in B73H

This site was assessed on a rapid level 3 and is situated in the KNP upstream of the Olifants Letaba confluence in vicinity of the existing Olifants_EWR16 site from the 2001 comprehensive Reserve study. The habitat during the survey was dominatedby sand and bedrock due to thevery low flows (Figure 29).

Figure 29: View of the EWR site on the Olifants River in B73H

The EWRfor the Olifants River in quaternary catchment B73H were determined for a REC of B/C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor October were also assessed as the surveys were undertaken on 15 October 2015. The discharge at the EWR site during the site visit was 2.964m3/s(Figure 30) and was used as reference to adjust the recommended EWRs.

Figure 30: Water levels on cross-section of the EWR site for the Olifants River in B73H

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The drought and maintenance low flows recommended by the DRM for October are 2.824 m3/s and 7.005 m3/s respectively, compared to the measured discharge of 2.964 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was not adequate to provide the necessary velocities and habitats as only sand and bedrock habitats will be available. Therefore, the DRM recommended flows were adjusted as follows:

Maintenance low flows for October: Adjusted from 7.005 m3/s to 8.318 m3/s

Drought flows for September: Adjusted from 2.638 m3/s to 3.424 m3/s

These requirements were also compared to the 2001 study results for a B category, with the maintenance low flows for October recommended as 7.0 m3/s and drought flows for September recommended as 2.0 m3/s. Although this study requirements are somewhat higher than the 2001 study results, it should be noted that the system changed substantially during the past 15 years and has formed constricted channels with limited habitats to sustain the biota.

The freshets and annual flood as specified for this site during the 2001 comprehensive study were assessed and some adjustments were made to derive the final results. These included changing from specifying freshets for drought years and freshets for maintenance years separately to only one set of freshets for ease of implementation. The initial and final freshets and floods for the Olifants River in B73H is provided inTable 44.

Table 44: Freshets and floods for the 2001 study and final requirements for implementation

| Months | 2001 Comprehensive study, Olifants_EWR16 |  |  |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Maintenance freshets |  | Maintenance floods |  | Drought freshets |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days | m3/s | days |
| October | 14 | 3 |  |  | 4 | 2 | 20 | 3 |
| November | 50 | 5 |  |  | 10 | 3 | 50 | 3 |
| December | 30 | 4 | 30 | 4 | 20 | 4 | 60 | 4 |
| January | 50 | 5 |  |  | 10 | 3 | 60 | 5 |
| February | 150 | 7 | 250 | 7 | 50 | 5 | 210 | 7 |
| March | 50 | 5 |  |  | 10 | 3 | 60 | 5 |
| April | 26 | 4 |  |  | 6 | 2 | 30 | 4 |

Table 45gives the various results of the DRM at the EWR site and the final EWR is summarised in Table 46.

Table 45: EWR results for specific months for the Olifants River in B73H (REC = B/C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 7.642 | 0.66 | 0.21 | 0.26 |
| High flows | February | 25.538 | 0.91 | 0.4 | 0.36 |
| Datum | October | 8.318 | 0.68 | 0.22 | 0.26 |
| Drought flows | September | 3.424 | 0.54 | 0.18 | 0.23 |
| Measured discharge at site visit (15 October 2015) |  | 2.964 | 0.513 | 0.18 | 0.225 |

Table 46: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B73H |
| --- | --- |
| River | Olifants |
| EWR Site Co-ordinates | S24.0494, E31.7318 |
| Recommended Ecological Category | B/C |
| VMAR at EWR site | 1 918.3 |
| Total EWR | 566.629 (29.54 %MAR) |
| Maintenance Low flows | 461.860 (24.08 %MAR) |
| Drought Low flows | 192.106 (10.01 %MAR) |
| Maintenance High flows | 104.769 ( 5.46 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_EWR5: Olifants River in B32D

This site is situated downstream Loskop Dam and was assessed as part of the 2001 comprehensive Reserve study and re-assessed during the development of the Reconciliation Strategy for the Olifants catchment in 2010. The Recommended Ecological Category for this site is a C. However, it was proposed during the Reconciliation Strategy that a minimum flow of 0.5 m3/s should be modelled and implemented.

This very low flow requirement was re-assessed using the existing hydraulics data from the 2001 study (see Figure 31) and it was found that it won’t provide adequate habitats and velocities for the fish and macroinvertebrates. Also, due to fish migration between Flag Boshielo Dam and Loskop Dam and to some of the smaller tributaries between the dams, additional freshets and floods were specified for specific months. The following recommendations were made as summarised in Table 47below.

Table 47: Recommended flows for EWR5 on the Olifants River in B32D (REC=C)

| Month | Maintenance flows | Drought flows | Freshets/ floods |  | Month | Maintenance flows | Drought flows | Freshets/ floods |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | m3/s | m3/s | m3/s | days |  | m3/s | m3/s | m3/s | days |
| Oct | 2.865 | 0.5 | 3 | 2 | Apr | 0.875 | 0.5 | 4 | 2 |
| Nov | 2.865 | 0.5 | 4 | 2 | May | 0.875 | 0.5 |  |  |
| Dec | 2.865 | 0.5 | 10 | 3 | Jun | 0.875 | 0.5 |  |  |
| Jan | 2.865 | 0.5 | 25 | 4 | Jul | 0.875 | 0.5 |  |  |
| Feb | 2.865 | 0.5 | 25 | 4 | Aug | 0.875 | 0.5 |  |  |
| Mar | 2.865 | 0.5 | 5 | 2 | Sep | 0.875 | 0.5 |  |  |

Figure 31: Water levels on cross-section of Olifants_EWR site for the Olifants River in B32D

The final EWR for a REC of C is summarised inTable 48 for the Olifants River in B32D.

Table 48: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B32D |
| --- | --- |
| River | Olifants |
| EWR Site Co-ordinates | S25.3040, E29.4220 |
| Recommended Ecological Category | C |
| VMAR at EWR site | 571.13 |
| Total EWR | 71.449 (12.51 %MAR) |
| Maintenance Low flows | 58.886 (10.31 %MAR) |
| Drought Low flows | 15.768 ( 2.76 %MAR) |
| Maintenance High flows | 12.563 ( 2.20 %MAR) |
| Overall confidence | Low to moderate |

## Olifants_EWR6: Lower Elands River in B31G

This site is situated downstream Mkhombo Dam and was assessed as part of the 2001 comprehensive Reserve study. No EWR was specified for the Lower Elands River during the development of the Reconciliation Strategy for the Olifants catchment in 2010, although a recommendation was made to address the current operation of Mkhombo Dam.

The Recommended Ecological Category for this site is a C/D and the existing hydraulics (Figure 32) from the 2001 study was used to make the following recommendations as summarised in Table 49 below.

Table 49: Recommended flows for EWR6 on the Elands River in B31G (REC=C/D)

| Month | Maintenance flows | Drought flows | Freshets/ floods |  | Month | Maintenance flows | Drought flows | Freshets/ floods |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | m3/s | m3/s | m3/s | days |  | m3/s | m3/s | m3/s | days |
| Oct | 0.183 | 0.07 | 0.5 | 2 | Apr | 0.183 | 0.07 |  |  |
| Nov | 0.183 | 0.07 |  |  | May | 0.183 | 0.07 |  |  |
| Dec | 0.183 | 0.07 | 2.5 | 2 | Jun | 0.07 | 0.07 |  |  |
| Jan | 0.183 | 0.07 | 5.0 | 3 | Jul | 0.07 | 0.07 |  |  |
| Feb | 0.183 | 0.07 | 5.0 | 3 | Aug | 0.07 | 0.07 |  |  |
| Mar | 0.183 | 0.07 |  |  | Sep | 0.07 | 0.07 |  |  |

Figure 32: Water levels on cross-section of Olifants_EWR6 site for the Elands River in B31G

The final EWR for a REC of C/D is summarised inTable 50for the Elands River in B31G.

Table 50: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B31G |
| --- | --- |
| River | Lower Elands |
| EWR Site Co-ordinates | S25.1160, E28.9565 |
| Recommended Ecological Category | C/D |
| VMAR at EWR site | 60.32 |
| Total EWR | 6.498 (10.77 %MAR) |
| Maintenance Low flows | 4.580 ( 7.59 %MAR) |
| Drought Low flows | 2.208 ( 3.66 %MAR) |
| Maintenance High flows | 1.918 ( 3.18 %MAR) |
| Overall confidence | Low |

## Olifants_EWR7: Olifants River in B51G

This site is situated downstream Flag Boshielo Dam and was assessed as part of the 2001 comprehensive Reserve study. No EWR was specified for this site during the development of the Reconciliation Strategy for the Olifants catchment in 2010. However, this is an important site for the releases of flow from Flag Boshielo Dam to ensure that the EWR as specified for the KNP can be met.

The Recommended Ecological Category for this site is a D and the existing hydraulics (Figure 33) from the 2001 study was used to make the following recommendations as summarised in Table 51 below.

Table 51: Recommended flows for EWR7 on the Olifants River in B51G (REC=D)

| Month | Maintenance flows | Drought flows | Freshets/ floods |  | Month | Maintenance flows | Drought flows | Freshets/ floods |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | m3/s | m3/s | m3/s | days |  | m3/s | m3/s | m3/s | days |
| Oct |  | 0.448 | 6 | 3 | Apr |  | 0.448 |  |  |
| Nov |  | 0.448 | 6 | 3 | May |  | 0.448 |  |  |
| Dec |  | 0.448 | 10 | 4 | Jun |  | 0.448 |  |  |
| Jan |  | 0.448 | 10 | 4 | Jul |  | 0.448 |  |  |
| Feb | 5.7 | 0.448 | 10 | 4 | Aug |  | 0.448 |  |  |
| Mar |  | 0.448 | 6 | 3 | Sep | 1.700 | 0.448 |  |  |

Figure 33: Water levels on cross-section of Olifants_EWR7 site for the Olifants River in BB51G

The final EWR for a REC of D is summarised inTable 52for the Elands River in B31G.

Table 52: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B51G |
| --- | --- |
| River | Olifants |
| EWR Site Co-ordinates | S24.5289, E29.5464 |
| Recommended Ecological Category | D |
| VMAR at EWR site | 726.64 |
| Total EWR | 117.302 (16.14 %MAR) |
| Maintenance Low flows | 109.059 (15.01 %MAR) |
| Drought Low flows | 14.128 ( 1.94 %MAR) |
| Maintenance High flows | 8.243 ( 1.13 %MAR) |
| Overall confidence | Low |

## Olifants_EWR9: Steelpoort River in B41H

This site is situated downstream De Hoop Dam and was assessed as part of the 2001 comprehensive Reserve study and re-assessed during the development of the Reconciliation Strategy for the Olifants catchment in 2010. The Recommended Ecological Category for this site is a D. It was proposed during the Reconciliation Strategy that the EWR as developed during the ORWRDP study be implemented for the Steelpoort River at EWR9.

However, due to the releases from De Hoop Dam, the system has improved and the REC determined during this study is a C category. Thus, the requirements were re-assessed using the hydraulics (Figure 34) from the 2001 study to determine the EWR for a C category. The results from the DRM for a C category was adjusted as follows:

Maintenance low flows for February: Adjusted from 1.030 m3/s to 1.503 m3/s

Maintenance low flows for September: Adjusted from 0.279 m3/s to 0.406 m3/s

No adjustments were made to the drought flow requirements. The following freshets and floods were specified as in Table 53.

Table 53: Recommended freshets/ floods for EWR9 on the Steelpoort River in B41H

| Month | Freshets/ floods |  |
| --- | --- | --- |
|  | m3/s | days |
| October | 2 | 2 |
| November | 5 | 2 |
| December | 10 | 3 |
| January | 15 | 3 |
| February | 20 | 5 |
| March | 10 | 3 |
| April | 5 | 2 |

Figure 34: Water levels on cross-section of Steelpoort River for Olifants_EWR9 site in B41H

The final EWR for a REC of C is summarised inTable 54for the Steelpoort River in B41H.

Table 54: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B41H |
| --- | --- |
| River | Steelpoort |
| EWR Site Co-ordinates | S24.7750, E30.1650 |
| Recommended Ecological Category | C |
| VMAR at EWR site | 137.5 |
| Total EWR | 37.665 (27.39 %MAR) |
| Maintenance Low flows | 26.675 (19.40 %MAR) |
| Drought Low flows | 9.150 ( 6.65 %MAR) |
| Maintenance High flows | 10.990 ( 7.99 %MAR) |
| Overall confidence | Low to moderate |

## LET2: Letaba River in B83D

This site was assessed on a rapid level 3 and is situated in approximately 4 km downstream of the existing Letaba_EWR7 site from the 2006 comprehensive Reserve study. The habitat during the survey was dominatedby sand and gravel with limited marginal vegetation due to the very low flows (Figure 35).

Figure 35: View of the EWR site on the Letaba River in B83D

The Ecological Water Requirements for the Letaba River in quaternary catchment B83D were determined for a Recommended Ecological Category of C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor October and February. October is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor the month of April were also assessed as the surveys were undertaken on 18April 2016. The discharge at the EWR site during the site visit was 0.530m3/s(Figure 36) and was used as reference to adjust the recommended EWRs.

Figure 36: Water levels on cross-section of the EWR site for the Letaba River in B83D

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The maintenance low flows recommended by the DRM for April and October are 2.439 m3/s and1.536 m3/s respectively, compared to the measured discharge of 0.530 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by themaintenance low flows for Octoberwas not adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows for a C category were adjusted as follows:

Maintenance low flows for October: Adjusted from 1.536 m3/s to 2.015 m3/s

The drought flow requirements as recommended by the DRM were accepted.

The freshets and annual flood as specified for this site during the 2006 comprehensive study were revised during the WRCS study in 2013. These were assessed and some adjustments were made to derive the final results. The initial and final freshets and floods for the Letaba River in B83D is provided inTable 55.

Table 55: Freshets and floods for the 2013 study and final requirements for implementation

| Months | 2013WRCS study, Letaba_EWR7 |  |  |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Class I |  | Class II |  | Class III |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days | m3/s | days |
| October |  |  | 15 | 4 |  |  |  |  |
| November |  |  | 15 | 4 |  |  | 5 | 2 |
| December | 3x6 | 3 | 15 | 4 |  |  | 10 | 2 |
| January | 6 | 3 | 15 | 4 |  |  | 15 | 2 |
| February |  |  |  |  | 120 | 6 | 80 | 4 |
| March |  |  |  |  |  |  | 20 | 2 |
| April | 6 | 3 | 15 | 4 |  |  | 10 | 2 |
| May |  |  |  |  |  |  | 5 | 2 |

Table 56gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 57.

Table 56: EWR results for specific months for the Letaba River in B83D (REC = C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | October | 2.015 | 0.19 | 0.11 | 0.4 |
| High flows | February | 4.960 | 0.27 | 0.17 | 0.53 |
| Datum | April | 3.200 | 0.22 | 0.14 | 0.45 |
| Drought flows | October | 0.731 | 0.12 | 0.06 | 0.32 |
| Measured discharge at site visit (18 April 2016) |  | 0.530 | 0.105 | 0.05 | 0.298 |

Table 57: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B83D |
| --- | --- |
| River | Letaba |
| EWR Site Co-ordinates | S23.8268, E31.5906 |
| Recommended Ecological Category | C |
| VMAR at EWR site | 646.28 |
| Total EWR | 112.049 (17.34 %MAR) |
| Maintenance Low flows | 89.672 (13.88 %MAR) |
| Drought Low flows | 31.748 ( 4.91 %MAR) |
| Maintenance High flows | 22.378 ( 3.46 %MAR) |
| Overall confidence | Low to moderate |

## LET14: Letsitele River in B81D

This site was assessed on a rapid level 3 and is situated just downstream of gauging weir B8H010 andupstream of the existing Letaba_EWR2 site from the 2006 comprehensive Reserve study. The habitat during the survey was dominatedby stones in and out of current and cobbleswith some boulders and bedrock present. The flows were moderate to high during the surveys (Figure 37).

Figure 37: View of the EWR site on the Letsitele River

The EWRfor the Letsitele River in quaternary catchment B81D were determined for a REC of D.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor the month of April were also assessed as the surveys were undertaken on 21 April 2016. The discharge at the EWR site during the site visit was 0.668m3/s(Figure 38) and was used as reference to adjust the recommended EWRs.

Figure 38: Water levels on cross-section of the EWR site for the Letsitele River in B81D

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The maintenance low flows recommended by the DRM for Apriland September are 0.399 m3/s and 0.193 m3/s respectively, compared to the measured discharge of 0.668 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by theDRM for maintenance low flows for September was not adequate to provide the necessary velocities and habitats. The following adjustments were made:

Maintenance low flows for September: Adjusted from 0.193 m3/s to 0.320 m3/s

The drought flow requirements as recommended by the DRM were accepted.

The freshets and annual flood as specified for this site during the 2006 comprehensive study were revised during the WRCS study in 2013. These were assessed and some adjustments were made to derive the final results.The initial and final freshets and floods for the lower Wilge River is provided inTable 58.

Table 58: Freshets and floods for the 2013 study and final requirements for implementation

| Months | 2013 WRCS study, Letaba_EWR2 |  |  |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Class I |  | Class II |  | Class III |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days | m3/s | days |
| November | 3.5 | 2 |  |  |  |  | 2.5 | 2 |
| December | 2x3.5 | 2 | 4.5 | 2 | 15 | 3 | 4 | 2 |
| January | 2x3.5 | 2 |  |  |  |  | 6 | 2 |
| February | 2x3.5 | 2 |  |  | 15 | 3 | 15 | 3 |
| March | 2x3.5 | 2 |  |  |  |  | 5 | 2 |
| April |  |  |  |  |  |  | 4 | 2 |
| May |  |  |  |  |  |  | 2.5 | 2 |

Table 59gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 60.

Table 59: EWR results for specific months for the Letsitele River in B81D (REC = D)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 0.320 | 0.26 | 0.11 | 0.35 |
| High flows | February | 0.777 | 0.35 | 0.16 | 0.46 |
| Datum | April | 0.662 | 0.33 | 0.14 | 0.431 |
| Drought flows | September | 0.193 | 0.22 | 0.08 | 0.31 |
| Measured discharge at site visit (21 April 2016) |  | 0.668 | 0.33 | 0.14 | 0.431 |

Table 60: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B81D |
| --- | --- |
| River | Letsitele |
| EWR Site Co-ordinates | S23.8932, E30.3576 |
| Recommended Ecological Category | D |
| VMAR at EWR site | 116.55 |
| Total EWR | 20.497 (17.59 %MAR) |
| Maintenance Low flows | 15.262 (13.09 %MAR) |
| Drought Low flows | 9.200 ( 7.89 %MAR) |
| Maintenance High flows | 5.236 ( 4.49 %MAR) |
| Overall confidence | Low to moderate |

## LET16: Great Letaba River in B81B

This site was assessed on a rapid level 3 and is situated upstream of the existing Letaba_EWR1 site from the 2006 comprehensive Reserve study. The habitats available during the survey was dominatedby stones in and out of current, cobblesand bedrock. Limited marginal vegetation was available. The flows were moderate during the site visit (Figure 39).

Figure 39: View of the EWR site on the Great Letaba River in B81B

The EWRfor the Great LetabaRiver in quaternary catchment B81B were determined for a REC of C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor October and February. October is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor the month of April were also assessed as the surveys were undertaken on 22 April 2016. The discharge at the EWR site during the site visit was 0.747m3/s(Figure 40) and was used as reference to adjust the recommended EWRs.

Figure 40: Water levels on cross-section of the EWR site for the Great Letaba River in B81B

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and depths for fish movement. The maintenance low flows recommended by the DRM for April and October are 0.719 m3/s and 0.398 m3/s respectively, compared to the measured discharge of 0.747 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for April and October was adequate to provide the necessary velocities and habitats. Therefore, the DRM recommended flows for a C category were accepted for the Great Letaba Wilge River in quaternary catchment B81B.

The freshets and annual flood as specified for this site during the 2006 comprehensive study were revised during the WRCS study in 2013. These were assessed and some adjustments were made to derive the final results.The initial and final freshets and floods for the Great Letaba River is provided inTable 61.

Table 61: Freshets and floods for the 2013 study and final requirements for implementation

| Months | 2013 WRCS study, Letaba_EWR1 |  |  |  |  |  | Final for implementation |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Class I |  | Class II |  | Class III |  | Final freshets/ floods |  |
|  | m3/s | days | m3/s | days | m3/s | days | m3/s | days |
| November | 2 | 2 | 3.5 | 3 |  |  | 2.5 | 2 |
| December |  |  | 3.5 | 3 | 8 | 4 | 4.5 | 2 |
| January |  |  | 3.5 | 3 |  |  | 10.5 | 2 |
| February |  |  | 3.5 | 3 | 20 | 6 | 25 | 3 |
| March | 2 | 2 | 3.5 | 3 |  |  | 15 | 3 |
| April |  |  |  |  | 8 | 4 | 4.5 | 2 |
| May |  |  |  |  |  |  | 2.5 | 2 |

Table 62gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 63.

Table 62: EWR results for specific months for the Great Letaba River in B81B (REC = C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | October | 0.398 | 0.36 | 0.2 | 0.29 |
| High flows | February | 0.748 | 0.455 | 0.27 | 0.372 |
| Datum | April | 0.719 | 0.45 | 0.27 | 0.37 |
| Drought flows | October | 0.223 | 0.29 | 0.16 | 0.24 |
| Measured discharge at site visit (22 April 2016) |  | 0.747 | 0.455 | 0.27 | 0.372 |

Table 63: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B81B |
| --- | --- |
| River | Great Letaba |
| EWR Site Co-ordinates | S23. 9178, E30.0507 |
| Recommended Ecological Category | C |
| VMAR at EWR site | 99.85 |
| Total EWR | 24.721 (24.76 %MAR) |
| Maintenance Low flows | 17.553 (17.58 %MAR) |
| Drought Low flows | 9.550 ( 9.56 %MAR) |
| Maintenance High flows | 7.168 ( 7.18 %MAR) |
| Overall confidence | Low to moderate |

## LET18: Broederstroom in B81A

This site was assessed on a rapid level 3 and is situated in the upper reaches of quaternary catchment B81A, upstream of Dap Naude Dam. The habitat during the survey was dominatedby small cobblesand gravel, mud and sand. Limitedstones in and out of current were available (Figure 41). The flows were moderate during the survey.

Figure 41: View of the EWR site on the Broederstroom

The EWRfor the Broederstroom in quaternary catchment B81A were determined for a REC of B/C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model.   The maintenance flows were examinedfor October and February. October is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor the month of April were also assessed as the surveys were undertaken on 22 April 2016. The discharge at the EWR site during the site visit was 0.192 m3/s(Figure 42) and was used as reference to adjust the recommended EWRs.

Figure 42: Water levels on cross-section of the EWR site for the Broederstroom in B81A

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for drought and maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates. The drought and maintenance low flows recommended by the DRM for October are 0.015 m3/s and 0.037 m3/s respectively and maintenance low flows of 0.068 m3/s for April, compared to the measured discharge of 0.192 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for October was not adequate to provide the necessary velocities and habitats. Therefore, the following adjustments were made:

Maintenance low flows for October: Adjusted from 0.037 m3/s to 0.058 m3/s

Maintenance low flows for September: Adjusted from 0.015 m3/s to 0.022 m3/s

The freshets and annual flood as specified for this site by the DRM was adjusted as in Table 64below.

Table 64: Freshets and floods requirements for implementation for the Broederstroom in B81A

| Months | Final for implementation |  |
| --- | --- | --- |
|  | Final freshets/ floods |  |
|  | m3/s | days |
| October | 0.5 | 2 |
| November | 0.5 | 2 |
| December | 0.5 | 2 |
| January | 0.6 | 2 |
| February | 1.9 | 2 |
| March | 0.6 | 2 |
| April | 0.6 | 2 |

Table 65gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 66.

Table 65: EWR results for specific months for the Broederstroom in B81A (REC = B/C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | October | 0.058 | 0.15 | 0.09 | 0.37 |
| High flows | February | 0.112 | 0.19 | 0.12 | 0.53 |
| Datum | April | 0.108 | 0.19 | 0.12 | 0.53 |
| Drought flows | October | 0.022 | 0.1 | 0.05 | 0.28 |
| Measured discharge at site visit (22 April 2016) |  | 0.192 | 0.242 | 0.15 | 0.549 |

Table 66: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B81A |
| --- | --- |
| River | Broederstroom |
| EWR Site Co-ordinates | S23.8011, E29.9772 |
| Recommended Ecological Category | B/C |
| VMAR at EWR site | 6.683 |
| Total EWR | 3.257 (49.22 %MAR) |
| Maintenance Low flows | 2.621 (39.61 %MAR) |
| Drought Low flows | 0.965 (14.58 %MAR) |
| Maintenance High flows | 0.636 ( 9.60 %MAR) |
| Overall confidence | Low |

## SHI1: ShingwedzI Riverin B90H

This site was assessed on a rapid level 3 and is situated in the lower reaches of the Shingwedzi River in the KNP in quaternary catchment B90H. The habitat during the survey was limited to some stones in current, gravel, sand and small cobbles (Figure 43). The flows were very low during the survey.

Figure 43: View of the EWR site on the Shingwedzi River

The EWRfor the Shingwedzi River in quaternary catchment B90H were determined for a REC of B/C.

The EWR flow data from the DRM was converted to hydraulic conditions at the EWR site (i.e. depths and flow velocities at discharges measured in m3/s) using a hydraulic model. The maintenance flows were examinedfor September and February. September is the month with the lowest maintenance flow (i.e. base-flow) and February is the month with the highest maintenance flow conditions (according to the DRM model).

The requirements of the DRMfor the month of April were also assessed as the surveys were undertaken on 19 April 2016. The discharge at the EWR site during the site visit was 0.006m3/s(Figure 44) and was used as reference to adjust the recommended EWRs.

Figure 44: Water levels on cross-section of the EWR site for the Shingwedzi River in B90H

Together with site photographs and rating relationships (flow depth versus discharge) from the hydraulic model, water levels proposed by the DRM for maintenance low flows were assessed in terms of the habitat and biotic requirements. The site-specific flow requirements were based mainly on the velocity and habitat requirements of flow-sensitive aquatic macroinvertebrates and movement of fish. The maintenance low flows recommended by the DRM for September and April are 0.001 m3/s and 0.107 m3/s respectively, compared to the measured discharge of 0.006 m3/s.

The consensus reached by the aquatic ecologists was that the velocities and availability of habitats provided by the DRM for these months wereadequate to provide the necessary velocities and habitats. Therefore, the DRM recommendations for maintenance low flows were accepted.

Due to the seasonal, almost ephemeral nature of the system, no drought flows were specified.

The freshets and annual flood as specified for this site by the DRM was accepted, except for January where the freshet was adjusted from 9.825 m3/s (2 days) to 19.650 m3/s (2 days).

Table 67gives the various results of the DRM at the EWR site and the final EWR is summarised inTable 68.

Table 67: EWR results for specific months for the Shingwedzi River in B90H (REC = B/C)

|  | Month | Discharge (m3/s) | Depth (m) |  | Velocity (m/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum | Average | Average |
| Maintenance low flows |  |  |  |  |  |
| Low flows | September | 0.001 | 0.03 | 0.02 | 0.03 |
| High flows | February | 0.729 | 0.32 | 0.19 | 0.16 |
| Datum | April | 0.107 | 0.17 | 0.10 | 0.07 |
| Measured discharge at site visit (19 April 2016) |  | 0.006 | 0.06 | 0.02 | 0.033 |

Table 68: Summary of the final EWR results (flows in million m3 per annum)

| Quaternary Catchment | B90H |
| --- | --- |
| River | Shingwedzi |
| EWR Site Co-ordinates | S23.1849, E31.5251 |
| Recommended Ecological Category | B/C |
| VMAR at EWR site | 86.424 |
| Total EWR | 19.449 (22.50 %MAR) |
| Maintenance Low flows | 5.541 ( 6.41 %MAR) |
| Drought Low flows | 0.000 ( 0.00 %MAR) |
| Maintenance High flows | 13.909 (16.09 %MAR) |
| Overall confidence | Low |

# RESULTS (QUALITY)

In terms of the water quality assessment, field survey and review the following was identified as key issues in the Olifants/Letaba system:

- High levels of salinity and related macro-ions

- Eutrophication – algal growth and water hyacinth in many parts of the catchment

- Microbiological pollution

- Discharges from mining, industries and wastewater treatment works

- Agricultural run-off

- Decants from mines – post closure

- Metal contamination (localised)

Table 69 provides an overview of the water quality state of the rivers in the Olifants/Letaba system with the associated land use impacts.

Table 69: Overview of water quality state and associated land use impacts in the rivers in the Olifants/Letaba System

| Olifants System: Less Impacted/Good Water Quality |  |
| --- | --- |
| River | Land use and Impacts/Water quality state |
| Mohlapitse | - Upper reaches good present water quality state. - Some silting - Contributes to quality of the Middle Olifants - Lower reaches before confluence with Olifants is being threatened by agricultural activities, cultivation and cattle grazing and trampling |
| Klip and Dwars tributaries in Steelpoort | - Good present state. |
| Upper Blyde | - Limited forestry and subsistence agriculture. - Good quality water - Good ecological condition |
| Olifants tributaries in KNP | - Tributaries are in good water quality condition - Important monitoring site for monitoring water quality and flow velocity to Mozambique in terms of international obligations. It also contributes to biodiversity protection in the Kruger National Park. |
| Olifants Sysem: Moderately Impacted Water Quality |  |
| Upper Elands | - The water quality is impacted by some algae and silt. - Agricultural activities - Settlements - Several small dams - Need to limit further deterioration |
| Kranspoortspruit | - Moderate water quality owing to limited upstream impacts - Need to ensure impacts are managed - Good ecological state |
| Steelpoort River | - The water quality is impacted by mining and sedimentation in lower reaches - Agricultural activities - Settlements - Need to limit further deterioration |
| Spekboom | - Land based activities impacting on water quality - Agriculture, mining, settlements, towns - Water is of acceptable quality. |
| Olifants upstream Blyde confluence | - Impacts - intensive citrus farming, game farming and subsistence grazing and cultivation and abstraction for commercial and subsistence farms. - Moderately impacted water but under threat |
| Lower Blyde | - Water quality is in a fairly good state - However under threate by extensive land based activities in the lower catchment area - Potentially reducing water quality entering into the lower Olifants River and thus into the Kruger National Park - Stringent management measures for the flows land based activities need to be implemented |
| Olifants | - 20km upstream from the Ga-Selati confluence and Phalaborwa impacts (Ga-selati River). - Localised impacts from irrigation - Sedimentation. |
| Olifants in KNP | - Water quality is a fairly good - Important for biodiversity protection in the Kruger National Park and international obligations to Mozambique |
| Olifants System: Poor Water Quality |  |
| Upper Olifants | - Poor water quality - Considerable algal growth (completely smothered) - Extensive coal mining, acid mine drainage - Town development and return flows from wastewater treatment works Witbank Dam and numerous small dams - High nutrients, salts - RQOs set for nutrients, salts, system variables and some toxins at EWR site, Witbank dam releases and Klipspruit |
| Klein Olifants | - Severe degradation of the system - upstream activities resulting in a high risk to the sustainability of the system. - Mining in upper catchment near Pullen Hopes /Arnot /Hendrina - Agricultural impacts - Untreated or poorly treated sewerage impacting on water resources |
| Upper Wilge | - Poor water quality - Bronkhorspruit – High algal growth and diatoms present (long stringy algae). |
| Lower Wilge | - Poor water quality - Impacted by organic pollution from agricultural activity, including mining activities. |
| Olifants – upstream Loskop | - Impacts (mining, industrial, agricultural, WWTWs) - Poor water quality |
| Selons | - High nutrient enrichment within the system |
| Middle Olifants | - Return flows, upstream activities namely mining, waste water treatment plant return flows, subsistence farming, agricultural activities all result in poor water quality |
| Ga-Selati | - Abstraction, farming, wastewater treatment works return flows, Phalaborwa industrial complex - Water quality is poor |
| Letaba River System: Water Quality State |  |
| Upstream Tzaneen Dam | - Good water quality - Some impact - WWTW, and sediment loads due to forestry activity |
| Thabina River | - Elevated nutrients, salts - Algal growth - WWTW discharges - Citrus plantations - Pesticide/fertilizer use |
| Letaba d/s Tzaaneen Dam / Tributaries | - Elevated nutrients, salts - Citrus plantations - Pesticide/fertilizer use |
| Brandboontjies River | - WWTW discharges - Elevated nutrients, salts - Algal growth |
| Upstream Middle Letaba Dam | - Extensive agricultural activities - Pesticide Herbicide use - Elevated nutrients, salts - Algal growth |
| Tributaries of Klein Letaba Tributaries of Letaba in KNP | - Good water quality |
| Klein Letaba River | - WWTW discharges - Elevated nutrients, salts - Algal growth |
| Shingwedzi Catchment: Water Quality State |  |
| Shingwedzi | - Mining impacts - Settlements impacting - Potential for deterioration in quality |
| Mphongolo | - Settlements having some impact on water quality - Land use activities - Water quality is acceptable |
| Shisa | - Good water quality |

Through the assessment and field surveys water quality hotspots/key areas that impact on the ecological condition of the rivers have been identified.  The specific sites that require water quality specifications to be set include:

- Olifants_S2 – Lower Wilge

- Olifants_S5 – Olifants in B11J

- Olifants_S6 – Klein Olifants

- Olifants_S10 – Olifants in B71D

- Olifants_S14 – Lower Blyde

- Olifants_S16 – Olifants in B73H

- LET2 – Letaba in B83D

- LET14 – Letsitele

- LET16 – Great Letaba in B81B

The RQOs set will determined will be applied were relevant, however where no RQOs exist or the RQOs do not include a water quality variable of importance, ecological specifications will be set to protect the water resource. In addition to this, strategic sub-catchment level water quality ecological specifications (at outlet nodes of catchment areas) will also be specified.

# CONCLUSIONS

The overall conclusion of this EWR report is that the present state of most of the tributaries as well as the main stem of the Olifants, Letaba and Shingwedzi Rivers have been degraded over the past few years since the previous comprehensive Reserve studies in 2001 and 2006.

However, it is acknowledged that the surveys were undertaken during a severe drought period (October 2015 and April 2016) with low to very low or no flows at the sites visited that provided limited habitats for fish and macroinvertebrates. The water quality was poor at most of the sites with no dilution capacity due to the low flows.

Drivers in the system contributing to the above, were both at a catchment scale (e.g. mining, waste water treatment works, limited/no releasesfrom the larger dams and agricultural activities impacting on flows and water quality) and localised (e.g. bank collapsing due to trampling and erosion).

Some specific sites that were impacted most are:

- Olifants River in B11J (Olifants_S5) showed a serious water quality problem with thick mats of algae as a result of Extensive coal mining, acid mine drainage, town development and return flows from wastewater treatment works and limited releases from Witbank Dam and numerous small dams

- Wilge River (Olifants_S2) that used to provide adequate flows of good quality was in a C category with a negative trend due to very low flows and poor water quality. Increased coal mining in the upper reaches of the Wilge, Saalboomspruit and Bronkhorstspruit and town developments with increased return flows from wastewater treatment works contributed to the poor water quality.

- Klein Olifants River shows a negative trend due to extensive mining in the upper catchment, agricultural impacts, Middelburg Dam with limited releases and untreated or poorly treated sewerage impacting on this water resource

- The flows in the middle reaches of the Olifants River from Flag Boshielo Dam to the border with KNP were very low. This is due to limited releases from Flag Boshielo Dam, De Hoop Dam and Blyderivierpoort Dam and extensive use for agriculture. Erosiion in most of this reach was high due to cattle trampling and over grazing.

- Lower Blyde River (Olifants_S14) showed a downward trend over the past few years due to extensive agricultural activities and limited releases from Blyderivierpoort Dam

- Olifants River in KNP (Olifants_S16) was assessed with a PES of a D category compared to a C category during the 2001 and 2010 assessments. This is mainly due to very low flows that can’t provide adequate habitats for the fish and macroinvertebrates.

- Great Letaba in B81B (LET16) showed a negative trend with a PES of a C/D category compared to the C category in 2006. This can be attributed to the extensive forestry, developments and alien invasive vegetation inthe riparian zone and limited releases from the upstream dams (Dap Naude, Ebenezer)

- Letaba River in KNP (LET2) showed the same negative trend as LET16. Most of the impacts that contributed to the PES of a C/D category are outside the KNP and include very low flows (large dams with limited or no flow releases) and nutrient enrichment as a result of the extensive agricultural activities outside the KNP.

- Shingwedzi River (SHI1) is an almost ephemeral system, with limited flows. However, the extensive use outside the KNP impacts on the flows at the EWR site that results in longer periods of very low of no flows.

The implementation of the EWRs as specified in this report, together with ongoing monitoring will assist in the sustainable management of the water resources of the Olifants, Letaba and Shingwedzi catchments.

# REFERENCES

DWA (2009). Olifants  River Water Resources Development Project (ORWDP): Phase 1 and 2.

DWA (2011). Development of a Reconciliation Strategy for the Olifants River Water Supply System. Report Number: P WMA 04/B50/00/8310/14. Department of Water Affairs, Pretoria, South Africa.

DWS (2014). A Desktop Assessment of the Present Ecological State, Ecological Importance and Ecological Sensitivity per Sub Quaternary Reaches for Secondary Catchments in South Africa. Department of Water and Sanitation (DWS), Pretoria, South Africa. Compiled by RQS-RDM.

DWAF (1999). Resource Directed Measures for Protection of Water Resources; Volume 3: River Ecosystems Version 1.0 (Revised Water Quality Methodology).

DWAF (2008): Methods for determining the water quality component of the Ecological Reserve. Report prepared for Department of Water Affairs and Forestry, Pretoria, South Africa by P-A Scherman. Draft 2, March 2008.

HUGHES, D.A. & MUNSTER, F (1999).  A decision support system for an initial “low confidence” estimate of the quantity component of the Reserve for rivers. Unpublished Report, Institute for Water Research, Rhodes University. pp. 32.

REPUBLIC OF SOUTH AFRICA (1998). National Water Act (Act No 36 of 1998).