ANNEXURE 8

strategic assessment of water re-use potential to augment the western cape water supply system

Summary Document

25 February 2009

Strategic Assessment of Water Re-use Potential to Augment the Western Cape

Water Supply System (WCWSS)

Summary Document

February 2009

# INTRODUCTION

Milkwood Communications was requested by DWAF, following a WCWSS Strategy Steering Committee (SSC) Meeting held on the 18 September 2008, to undertake a strategic assessment of the water re-use potential within the WCWSS, based on the City of Cape Town’s effluent discharges, and to propose a way forward to utilise this potentially valuable resource.  The results were to be presented at the next SSC meeting scheduled for 11 March 2009.  This summary document highlights the main findings of the study and gives a brief overview of the type of information that will be contained in the final report.  Much of the detail has been left out of this document, but will appear in the final report to substantiate all the conclusions and recommendations.

# Terms of Reference of Strategic Assessment

The purpose of the study was to assess the potential for the re-use of water (treated effluent) as a water augmentation option for the City of Cape Town, specifically as an extension to the existing Western Cape Water Supply System (WCWSS).  The study consisted of the following components:

- A review of previous water re-use studies undertaken by the CCT.

- A review of the latest information available on water re-use practice nationally and internationally.

- Obtaining up-to-date information on the availability and quality of treated effluent, as well as existing and potential re-use schemes.

- Identify barriers to implementation of water re-use schemes and put forward strategies to address these barriers.

- Conceptualise potential water re-use schemes and do preliminary costings for them.

- Formulate a recommended strategic approach towards water re-use for augmentation of the WCWSS.

This report briefly covers each of these components, with the most emphasis being placed on the conclusions and recommendations.

# Categories of WATER Re-use

Water can be re-used in a number of different ways.  Table 1 summarises the three main categories of water re-use.

Table 1	Categories of water re-use

| Planned Use is pre-planned to occur through an engineered system | Potable Use for potable purposes, e.g. drinking water | Direct Treated effluent returned directly into the potable water supply system |
| --- | --- | --- |
| Unplanned Effluent is discharged into a river or water body and abstracted downstream for potable water supplies. Common worldwide. | Non-potable Use by industry, irrigation, domestic dual reticulation | Indirect Treated effluent returned into a receiving water body, e.g. dam, lake or aquifer, from where the water is abstracted again and treated for potable water supply |

Many water supply projects around the world actually fall into the unplanned indirect potable re-use category, since towns in the upper reaches of catchments discharge treated effluent into the river, and towns downstream abstract water from the river, treat it, and supply it as potable water.  The Thames River in England is often quoted as an example of this, but the Vaal River in South Africa is another example.

# Review of Previous Studies

Water re-use was studied as part of numerous different projects undertaken for the City of Cape Town and DWAF over the years.  The studies, as well as a brief description of what each study entailed, are given in Appendix A.

The main conclusion from this review of past studies is that in each case the re-use of water was found to have significant potential as a water resource, and in each case it was recommended that further studies be done to investigate this potential in more detail.  Resource constraints and institutional change have made it difficult for a number of the recommendations to be implemented by the responsible local authority.

# Review of national and international practice

The latest information available on water re-use practices nationally and internationally were reviewed, and a number of examples of water re-use are given below.

## Case Studies

Projects that have been implemented in South Africa and internationally were identified and assessed as to how they are performing.  The major water re-use schemes in operation around the world today are listed in Table 2 below.  Detailed descriptions of these projects will be given in the final report.

Table 2	Case studies of water re-use

| Scheme | Implementation Date | Category of re-use | Description |
| --- | --- | --- | --- |
| Scottsdale Water Campus, Arizona, USA | 1999 | Planned indirect non-potable and potable | Water Campus consists of a 45 Ml/d WT plant for non-potable use, and a 38 Ml/d advanced WT plant for groundwater recharge for potable use |
| Upper Occoquaan Sewage Authority, Washington DC, Virginia | 1978 | Planned indirect potable | Conceived as a water quality improvement programme for Occoquan Reservoir, which is the principal water source for >1 million people. Capacity of scheme is 120 Ml/d. Occoquan Watershed Monitoring Laboratory was established at the outset. |
| Orange County, California | 1976 (current upgrade approved 2003 – completed 2007) | Planned indirect potable | Effluent treated at Water Factory 21 and used for aquifer recharge and as a seawater barrier. Capacity is 63 Ml/d and supplies 2 million people |
| Queensland’s Western Corridor Recycled Water Project, Brisbane, Australia | starting 2008 | Planned indirect potable and non-potable re-use | Supply to industry, power station cooling water, and Lake Wivenhoe, the region’s principal storage reservoir, from which water is abstracted for potable supplies. Three advanced Water Treatment plants with a total capacity of 232 Ml/d. About half will go to indirect potable use. |
| The Montebello Forebay Ground Water Recharge Project, Los Angeles, California, USA | 1962 | Planned indirect potable re-use | Recharge to an aquifer |
| Hueco Bolson Recharge Project, El Paso, Texas, USA | 1985 | Planned indirect potable re-use | Supply to 250 000 people |
| Singapore's NEWater, Singapore | 2003 | Planned indirect potable and non potable re-use | Supply to industry and a dam. 1% of drinking water to 4.2 million people. |
| Atlantis, South Africa | Stormwater recharge – 1980. Treated effluent recharge – 1987. | Planned indirect potable re-use | Recharge 4.5 Ml/d of treated effluent to the aquifer. 15 Ml/d is abstracted from this aquifer to supply 70 000 people. In addition, treated industrial wastewater is used as a barrier against saltwater intrusion near the coast. |
| Roodeplaat Dam, City of Tshwane, South Africa | 2005 | Planned indirect potable re-use | Supply to a dam |
| Windhoek, Namibia | 1968 | Planned direct potable re-use and planned indirect potable re-use | Direct supply to water supply system and recharge to an aquifer. 26% - 36% of drinking water, supply to 240 000 people. |

Figure 1 below shows the various major milestones in water re-use since the Windhoek direct re-use scheme started operation in 1968, including the various technological advances that have been made since then.

Figure 1:	Salient milestones in advanced re-use applications (after Law, 2003)

The various milestones relevant to Southern Africa include the construction of the Windhoek plant in 1968 using the technology of that era, although the plant was continuously updated as technology improved over the years.  Ozone coupled with activated carbon was first trialed in a water reclamation context in 1978 at the 5,000 m3/day Stander Plant in Pretoria.

The improvements in technology are exemplified by the comment by Law (2003) that it is unlikely that the treatment train that was initially implemented at Windhoek in 1968 at the Old Goreangab Water Reclamation Plant will ever be used again for potable re-use; it was considered appropriate at the time but would fall far short of acceptance today.  There have been four technology changes/upgrades at Windhoek since1968, with the most recent being in 2000 when, amongst other changes, an ultra-filtration (UF) membrane filtration system was installed.

## Best Practice

The publication entitled “Best Practices for Indirect Potable Re-use: Phase 1 Report” by the Water Re-use Association (2004) was obtained from the USA.  The goal of the study was to identify best practices to ensure that well planned indirect potable re-use projects receive fair consideration in water supply decisions.  Marketing, communication and technical professionals were brought together and came up with 25 best practices.

Six case studies were chosen to compare against this list of best practices.  Three of the case studies were successfully implemented, and are the first three examples listed in Table 5.1 (Orange County California Groundwater Recharge System (USA), Upper Occoquan Sewage Authority (Virginia, USA) and Scottsdale (Arizona, USA)).  The other three case studies were not implemented due to negative public opinion.  Each case study was compared against the 25 best practices, and the best practices refined accordingly.  The list of best practices and excerpts from the publication are given in Appendix B.

A number of the best practises relating to implementation risk are highlighted below:

Risk-based best practices

Understand and avoid environmental justice issues

"Don't use minority groups or previously disadvantaged groups as "guinea pigs".  Ideally the leaders proposing the project should be in the community that is being supplied.  Don't offer lower rates for the water."

Understand and use track records

The track record of the local water agency should be used to build confidence that the water quality will continue to be of a high standard should the water re-use project be implemented.  The health and safety records of existing water re-use projects can also be cited.

Break the source-quality connection

This can be achieved by using multiple barriers (i.e. ultra filtration, reverse osmosis and UV radiation) and by ensuring a redundancy of barriers.  An example of this is returning the purified water back to a water resource and then ultimately treating the raw water again by means of a conventional treatment process.  The public should be made aware that the water being returned to the resource will be of a higher quality than the existing water in the resource.

For example: this study was originally called a "Strategic Assessment for the Use of Treated Effluent in the City of Cape Town", but the title of the report has been changed to exclude the words "Treated Effluent", as a first step in this recommendation.

Articulate an ongoing Water Quality Plan

"The public will be more confident if they are shown a plan for increasing knowledge about risks and improving quality over time, as well as an emergency response plan.  These will show that the diligence of the water utility is what ensures quality and safety, not the physical source of the water."

Other best practises would include the following:

Communicate all the benefits of indirect potable re-use

"Community leaders will support or consent to indirect potable re-use if they believe that the benefits greatly outweigh the risk.  Indirect potable re-use will have to be accurately positioned against the other alternatives in order for it to get fair consideration.  The benefits can be stated in terms of the problems that it solves:

- underutilisation of water

- climate dependency of water supplies

- need for investment in separate infrastructure

- underutilisation of “potable” storage assets

- “unplanned” indirect potable re-use is already happening

- increases surface water quality

Water quality is dependent more on treatment and monitoring and less on the physical source of the water.  Indirect potable re-use projects typically (and should) lead to higher water quality."

Rename the water quality

"The terms "recycled water" or "reclaimed water" do not have to be used to describe a project.  These terms emphasise the source of the water.  "Recycled" implies "re-used".  How many people buy re-used clothes or re-used tyres?  Re-used, recycled and reclaimed imply second class, not the best of class.  Choose another term such as "purified", "highly purified" or simply drinking water to convey the higher quality of the water.  The term purified will be more easily accepted if processes such as reverse osmosis are being proposed and installed.  Disapproving slogans can create a negative idea of value.  Agencies should consider creating a new idea of value to associate with the new name.  In the Scottsdale case study, the water department effectively branded the wastewater effluent as a "valuable resource".

Singapore has used the term "NEWater".

## Conclusions and Recommended Approach

Based on the literature review undertaken, it is clear that international best practice is to focus on indirect planned re-use for potable and non-potable purposes.  The only place in the world that currently practices direct potable re-use is Windhoek, Namibia, and even there it is thought unlikely that approval to implement the scheme would be obtained if they tried to initiate the scheme now (pers comm. P du Pisani  (City Engineer, Windhoek), February 2009).

It is therefore proposed that, for augmenting the supply to the WCWSS, indirect potable and non-potable schemes be investigated.  As a result of this approach, and to conform to international best practice, no new direct potable options were conceptualised as part of this strategic investigation.  The costs of the direct potable scheme put forward in a previous study (Reconciliation Strategy Study) were updated for comparison purposes only.  It is not proposed that this option be investigated further.

# Current Status of re-use in Cape Town

## Volume of effluent available

There are 16 wastewater treatment works (WWTW) in the City of Cape Town, as well as a number of marine outfalls, the locations of which are shown on Figure 2.  The effluent disposed of via the marine outfalls is not of a suitable quality for re-use, so was not considered in this study.

Figure 2:	Location of the CCT’s WWTWs

Approximately 600 Ml/d (218 Mm3/a) of treated effluent was discharged from the WWTWs (excluding marine outfalls) in 2007/2008.  Most of the effluent (90%) is discharged from the 8 largest works, with about half (53%) being treated at the two largest WWTW, namely Cape Flats and Athlone.  The volumes discharged for each works are given in Table 3.  This study focuses on the effluent discharged from these eight larger works (Cape Flats, Athlone, Zandvliet, Bellville, Mitchells Plain, Macassar, Potsdam, Borcherds Quarry).

Table 3	Volume of effluent treated at WWTW

| Name | Capacity of WWTW |  | Actual Flows 2007/08 |  |
| --- | --- | --- | --- | --- |
|  | Capacity of WWTW | Capacity of WWTW | Average Annual Daily Flow | Total Annual Flow |
|  | Ml/d | Mm3/a | Ml/d | Mm3/a |
| Cape Flats | 200 | 73.0 | 172.5 | 63.0 |
| Athlone | 120 | 43.8 | 119.3 | 43.6 |
| Zandvliet | 55 | 20.1 | 59.5 | 21.7 |
| Bellville | 46 | 16.8 | 52.3 | 19.1 |
| Mitchells Plain | 37.5 | 13.7 | 33.6 | 12.3 |
| Macassar | 34 | 12.4 | 31.3 | 11.4 |
| Potsdam | 32 | 11.7 | 35.1 | 12.8 |
| Borchards Quarry | 30 | 11.0 | 33.0 | 12.1 |
| Wesfleur (Atlantis) | 14 | 5.1 | 9.5 | 3.5 |
| Wildevoëlvlei | 14 | 5.1 | 11.6 | 4.2 |
| Kraaifontein | 7 | 2.6 | 20.5 | 7.5 |
| Simonstown | 5 | 1.8 | 2.3 | 0.8 |
| Scottsdene | 4.5 | 1.6 | 9.8 | 3.6 |
| Gordons Bay | 3.5 | 1.3 | 3.3 | 1.2 |
| Melkbos Strand | 2.5 | 0.9 | 3.3 | 1.2 |
| Parow | 1.2 | 0.4 | 0.8 | 0.3 |
|  | 606.2 | 221.3 | 597.8 | 218.2 |

## Seasonal availability of effluent

The seasonal distribution of the effluent discharged is shown in Figure 3.  The bulk of the effluent is available in the winter months (especially June, July, August), due to stormwater ingress into the sewers.  Therefore to maximise the use of the treated effluent resource, this study focuses on all year round use.

Figure 3:	Seasonal distribution of treated effluent

## Effluent quality

A number of sets of standards for treated effluent are relevant to the effluent discharged by the WWTW in Cape Town.  The existing General and Special Standards that were published in 1984 currently apply, and the updated (more stringent) standards will be applied by DWAF from 2010. The City does not comply with the current standards for effluent on a consistent basis.  This can be attributed to the financial, operational and technical constraints being experienced by the CCT.  As was discussed in Section 5.3 of this document, this study is proposing that only indirect re-use options are considered, and that advanced treatment processes such as reverse osmosis (RO) are used.  This level of processing will deal with most potential contaminants that may still be in the treated effluent, so for the purposes of this study, effluent quality is not of strategic importance.

Quality constraints are, however, relevant for the local re-use proposed as part of the CCT’s Effluent Re-use Master Plan, and need to be addressed in order to maximise the potential identified in the Master Plan.  According to BVI Consulting Engineers, it seems that there would be a demand for treated effluent of a slightly better quality, even if the cost per kl was higher than the current rate.

## Current and planned future re-use of treated effluent in Cape Town

Currently, approximately 80.5 Ml/d (21.1 Mm3/a) of treated effluent is used (10% of the total annual volume available).  Of this, it is estimated that approximately 32.8 Ml/d (7.3 Mm3/a, which is 35% of the annual volume re-used) replaces potable use; i.e. would contribute towards demand reduction.

The CCT has a Master Plan (CCT, 2007) in place for treated effluent use, and this has identified an additional 74.8 Ml/d (10.3 Mm3/a) from April 2007.  Approximately 56.5 Ml/d (7.0 Mm3/a, which is 68% of the annual volume of this) replaces existing potable use.

This gives a total future re-use of 155.3 Ml/d (31.4 Mm3/a) (14% of the total annual volume currently available).  Of this, it is estimated that 89.3 Ml/d (14.3 Mm3/a, or 46% of the annual volume) will replace existing potable water use in the City.  The estimates of existing and potential re-use volumes for each WWTW are given in Table 4.

Table 4:	Estimated current and potential future re-use per WWTW

| WWTW | Existing daily re-use |  | Potential additional re-use |  | Total potential annual re-use |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total | Portion replacing potable | Total | Portion replacing potable | Total | Portion replacing potable |
|  | Mm3/a | Mm3/a | Mm3/a | Mm3/a | Mm3/a | Mm3/a |
| Athlone | 0.648 | 0.000 | 1.775 | 1.609 | 2.423 | 1.609 |
| Bellville | 1.995 | 1.019 | 2.625 | 1.145 | 4.620 | 2.164 |
| Borchards Quarry | 0.360 | 0.000 | 0.000 | 0.000 | 0.360 | 0.000 |
| Cape Flats | 1.305 | 0.000 | 0.472 | 0.472 | 1.777 | 0.472 |
| Gordons Bay | 0.100 | 0.000 | 0.133 | 0.133 | 0.233 | 0.133 |
| Kraaifontein | 2.710 | 0.226 | (0.317) | (0.317) | 2.393 | (0.091) |
| Macassar | 0.810 | 0.000 | 0.326 | 0.326 | 1.136 | 0.326 |
| Melkbos Strand | 0.396 | 0.000 | (0.036) | (0.036) | 0.360 | (0.036) |
| Mitchells Plain | 0.000 | 0.000 | 0.781 | 0.636 | 0.781 | 0.636 |
| Parow | 0.259 | 0.039 | 0.049 | 0.049 | 0.308 | 0.088 |
| Potsdam | 8.276 | 4.553 | 3.387 | 2.289 | 11.663 | 6.842 |
| Scottsdene | 2.034 | 0.062 | 0.200 | 0.200 | 2.234 | 0.262 |
| Wesfleur (Atlantis) | 1.680 | 1.407 | 0.196 | 0.196 | 1.876 | 1.603 |
| WildeVoëlvlei | 0.000 | 0.000 | 0.679 | 0.305 | 0.679 | 0.305 |
| Zandvliet | 0.540 | 0.000 | 0.035 | 0.035 | 0.575 | 0.035 |
|  | 21.113 | 7.307 | 10.305 | 7.041 | 31.418 | 14.348 |

Most of the existing and potential re-use takes place in summer.  Of the current volume of effluent re-used (21.1 Mm3/a), approximately 77% (16.38 Mm3/a) is estimated to be summer re-use, and the remaining 23% (4.9 Mm3/a) is estimated to be winter re-use.

The Master Plan set a limit on the cost of supplying treated effluent of R2.50/kl, which is approximately half the price of potable water.

## Surplus effluent available

It is estimated (Reconciliation Strategy Study) that the volume of effluent which would be discharged in the year 2020 will be 705 Ml/d (257 Mm3/a).  Assuming the Master Plan will be fully implemented by then, the total estimated surplus effluent available after proposed re-use in 2020 is 549 Ml/d (226 Mm3/a).  Most of this surplus is available in winter.  Table 5 contains details of these estimates.  Pie charts showing the surplus effluent available are given in Figure 2.

Table 5:	Estimated surplus effluent available per WWTW in 2020

| Name | Future flow estimates |  | Total existing and potential re-use |  | Surplus effluent 2020 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Projected annual daily average flow 2020 | Projected total annual flow 2020 | Average annual daily flow | Total annual flow | Average annual daily flow | Total annual flow |
|  | Ml/d | Mm3/a | Ml/d | Mm3/a | Ml/d | Mm3/a |
| Athlone | 120.5 | 44.0 | 15.3 | 2.423 | 105 | 41.6 |
| Bellville | 68.5 | 25.0 | 19.5 | 4.620 | 49 | 20.4 |
| Borchards Quarry | 49.3 | 18.0 | 2.0 | 0.360 | 47 | 17.6 |
| Cape Flats | 178.1 | 65.0 | 16.1 | 1.777 | 162 | 63.2 |
| Gordons Bay | 4.7 | 1.7 | 2.0 | 0.233 | 3 | 1.5 |
| Kraaifontein | 17.0 | 6.2 | 9.0 | 2.393 | 8 | 3.8 |
| Macassar | 64.9 | 23.7 | 11.1 | 1.136 | 54 | 22.6 |
| Melkbos Strand | 4.1 | 1.5 | 2.2 | 0.360 | 2 | 1.1 |
| Mitchells Plain | 35.1 | 12.8 | 6.1 | 0.781 | 29 | 12.0 |
| Parow | 2.9 | 1.1 | 1.9 | 0.308 | 1 | 0.7 |
| Potsdam | 52.9 | 19.3 | 44.6 | 11.663 | 8 | 7.6 |
| Scottsdene | 18.0 | 6.6 | 8.3 | 2.234 | 10 | 4.3 |
| Wesfleur (Atlantis) | 13.7 | 5.0 | 6.4 | 1.876 | 7 | 3.1 |
| Wildevoëlvlei | 15.3 | 5.6 | 4.8 | 0.679 | 11 | 4.9 |
| Zandvliet | 57.0 | 20.8 | 6.0 | 0.575 | 51 | 20.2 |
| Simonstown | 2.8 | 1.0 | 0.0 | 0.000 | 3 | 1.0 |
|  | 704.8 | 257.2 | 155.3 | 31.418 | 549 | 225.8 |

Of the eight largest works, treated effluent from Potsdam will be almost fully utilised by 2020, so this works was excluded from further consideration in this study.  The estimated surplus from the remaining 7 larger works is approximately 545 Ml/d (200 Mm3/a).  This volume was considered to be available when conceptualising and costing the various potential schemes in this study.

# Potential Schemes (costings)

The potential schemes put forward as part of this study are listed in Table 6 in the appropriate categories of re-use.

Table 6:	Potential Water Re-use Schemes put forward as part of this study

| Category of re-use |  | Potential re-use scheme |  |
| --- | --- | --- | --- |
| Planned direct | non-potable | CCT’s Master Plan – re-use by industry and urban irrigation |  |
|  | potable | Blending option costed in Reconciliation Strategy Study (UWP) (integrated into WCWSS) |  |
| Planned indirect | non-potable | Berg River irrigation exchange |  |
|  |  | Irrigation Exchange (Stellenbosch) |  |
|  | potable | Discharge into existing dams / aquifers and integrate with the WCWSS | Steenbras Dam – existing |
|  |  |  | Steenbras Dam – raised |
|  |  |  | Berg River Dam |
|  |  |  | Cape Flats Aquifer recharge |
|  |  | Discharge into new dam (not costed) | Diep River (Platrug Dam) |
| Benchmark against desalination (integrated into WCWSS) |  |  |  |

A brief description of each potential scheme is given below.  The location of the development options is shown on Figure 4.

Figure 4:	Potential Water Resource Development Options

## Scheme 1: Delivery of Effluent from WWTW along False Bay Coast to Upper Steenbras Dam

This development option is based on using effluent produced by the four WWTWs located along the False Bay coast (Cape Flats, Macassar, Mitchells Plain and Zandvliet).  This effluent would be treated at a reverse osmosis plant located near the Macassar WWTW and purified to potable standard.  It is estimated that by the year 2020, approximately 118 Mm3/a of effluent would be available for treatment.  Approximately 30% of the inflow to the plant would be lost in the purification process, therefore supplying approximately 83.2 Mm3/a of water before conventional treatment.  The purified water would be pumped into the Upper Steenbras Dam at a point some 2.5 km upstream of the dam wall.  The water would blend with the natural runoff stored in the dam before being abstracted via the existing Steenbras Pumped Storage Scheme and pumped to the existing Faure and Blackheath Water Treatment Works for normal treatment and distribution to consumers.

The unit reference value of this scheme at a 6 % discount rate is estimated to be R5.80.

A variation of this Scheme was also costed which included the cost of raising of Lower Steenbras Dam.   A raised Lower Steenbras Dam would also deliver an additional yield of approximately 20 Mm3/a.  This additional yield was, however, ignored in the URV calculations, as additional pipelines would then also be required between Upper Steenbras Dam and Faure Water Treatment Works.  The investigation into the raising of the Lower Steenbras Dam is going to be carried out as part of DWAF's Western Cape Feasibility Study of various supply interventions.

The unit reference value of this scheme, with a raised Lower Steenbras Dam, at a 6% discount rate, is estimated to be R6.00.

## Scheme 2: Delivery of Effluent from WWTW along False Bay Coast to  the Berg River Dam

This development option is similar to Scheme 1 in that it is based on using effluent produced by the four WWTWs located along the False Bay coast, but pumps the purified water into the Berg River Dam.  This would be done through a new pipeline routed via Stellenbosch and Simondium in order to avoid the mountains.  The pipeline would discharge into the dam about 2 km upstream of the dam wall.  The water would blend with the natural runoff stored in the dam before being abstracted as part of the normal supply and distributed to the water treatment works supplied from the Berg River Dam, for normal treatment and distribution to consumers.

The unit reference value of this scheme at a 6% discount rate is estimated to be R5.80.  This excludes any additional cost of having to pump the additional water from the Berg River Dam to Theewaterskloof Dam.

## Scheme 3: Recharge of Cape Flats Aquifer

This development option would have a lower yield than Schemes 1 and 2.  It is also based on using the effluent produced by the four WWTWs located along the False Bay coast, but only the summer flow would be used in this option.  It is proposed to pump 50 Mm3 in the period from the beginning of November to the end of April the next year to a reverse osmosis treatment plant located near the Mitchells Plain WWTW.  The quantity of 50 Mm3 is the estimated total quantity of effluent that will be available during the summer months from the four treatment works by 2020.  After losses in the reverse osmosis process, about 34 Mm3/a purified water would be available.  This would be injected into the Cape Flats Aquifer in the area to the west of the Mitchells Plain residential area.  The water would be injected during the summer months only because the water table is naturally close to the surface during the wet winter months.  Water would be abstracted from the aquifer throughout the year by means of boreholes spread throughout the Mitchells Plain residential and surrounding area, and pumped to the Faure Water Treatment Works for conventional treatment and distribution to consumers.

The combination of effluent recharge and natural groundwater is expected to give a sustained yield of about 52 Mm3/a.

The unit reference value of this scheme at a 6% discount rate is estimated to be R4.20/m3.

## SEAWATER Desalination

In order to be able to benchmark the planned indirect use of potable water with desalination of seawater, a scheme was conceptualised to integrate a large-scale desalination plant into the bulk water supply infrastructure of the WCWSS. A 228 Ml/d desalination plant to be situated next to the Koeberg Power Plant was costed. The desalinated water was pumped to a 300 Ml/d bulk water supply reservoir (Koeberg Hill) at an elevation of approximately 180 masl. The bulk reservoir was linked to the Voëlvlei bulk water pipeline in order to be able to blend the desalinated water with treated water prior to distribution. The proposed reservoir was also linked to the proposed Voëlvlei to Glen Garry Scheme in order to ensure that the new bulk water supply could feed the area’s high growth.

The unit reference value of this scheme, at a 6% discount rate, is estimated to be between R12.00 and R13.00, depending on the infrastructure required to integrate the desalination plant into the WCWSS.

## Summary of Costs

The results of the preliminary costing exercise undertaken are summarised in Table 7 where unit reference values (URVs) for each scheme are given.  The updated URVs will be contained in the main report.

Table 7:	Comparison of URVs of Development Options

| Options | Potential yield of scheme (Mm3/a) | URV (real elec. increase) | URV (no elec. increase) |
| --- | --- | --- | --- |
| Return treated effluent to Steenbras | 83 | 5.8 | 4.3 |
| Return treated effluent to Steenbras (raised) | 83 | 6.0 | 4.5 |
| Return treated effluent to Berg River Dam | 83 | 5.8 | 4.4 |
| Recharge Cape Flats Aquifer | 83 | 4.5 | 3.5 |
| Desalination | 83 | 11.9 | 6.9 |
| Desalination (into system) | 83 | 13.1 | 7.7 |
| Irrigation exchange (Stellenbosch) | 5 |  | 3.0 |
| Planned direct potable use - Faure | Still to be confirmed |  |  |

## Conclusions

The following conclusions can be drawn from the investigation into the comparison of the development options:

- The development option where the treated water is pumped back to Steenbras Dam is technically feasible, but would place limitations on the operation and optimisation of the water resources within the WCWSS. The primary reason for this is that the additional water which is pumped into Steenbras Dam has to be pumped back to Faure and utilised to supply winter demand, as no additional storage capacity is provided in Steenbras Dam.

- Pumping the treated water into the Berg River Dam or into a raised Lower Steenbras Dam would both be technically feasible options and would also not impact significantly on the operation of the WCWSS.

- The cost of planned indirect use of treated water for potable supply is significantly cheaper than the desalination of sea water.

- The cost and URVs of all of the development options are sensitive to real electricity tariff increases.  The reason for this is that both desalination and the planned indirect use of treated effluent are energy intensive processes.

- Desalination of seawater is especially sensitive to real electricity tariff increases. This is due to the high TDS of seawater and the high pressure required for the reverse osmosis process.

- The development options proposed need to be modelled in the system model in order to ensure that the yields proposed are realised and that the proposed development options do not negatively impact on the operation and optimisation of the water resources in the WCWSS.

# Recommended strategic approach for the future use of treated effluent

The overall conclusion which can be drawn from this strategic investigation is that significant potential exists for the large-scale re-use of water in the City of Cape Town, and that this resource can be integrated into the WCWSS.

The main recommendations are given below.

## Limit development options to planned indirect potable and non-potable re-use

In line with international best practice, it is recommended that only indirect potable and non-potable re-use development options are pursued.  Initial cost estimates for these indirect re-use options are promising, indicating that planned indirect re-use of water may compare favourably against other water resource development options.  The planned indirect use of water is significantly cheaper than desalination.

## Feasibility study

It is recommended that CCT, in conjunction with DWAF, initiate a feasibility study on indirect planned re-use and the potential for re-use as a water augmentation scheme for the WCWSS.  The results of this feasibility study should be compared with those of the feasibility studies currently underway, such as the Michell’s Pass Diversion, Voëlvlei Phase 1, etc.

## Best Practices

Recommendations from the publication on “Best Practices for Implementing Indirect Potable Re-use Projects” should be implemented, specifically:

- Create a perception of improvement

- Communicate all the benefits of indirect potable water re-use

- Understand and avoid environmental justice issues

- Understand and use track records

- Break source-to-tap connection

- Establish the Water Agency and investment as the sources of water quality (including the establishment of a separate monitoring body)

- Rename the water quality.

## Local Re-use

For those WWTWs that are not candidates for supply to a large development option, local re-use options should be maximised.  The focus for local re-use should be on those users who require year-round supplies as well as where the re-use would replace existing potable water use.  The potential and cost-effectiveness of localised post-treatment to provide the required quality of water should also be investigated.  This would involve the review of the threshold charge of R2.50/kl for treated effluent by the CCT.

# References

Du Pisani, P.  Pers comm..  (City Engineer, Windhoek), February 2009.

CCT, 2007.  Treated Effluent Reuse Strategy and Master Planning Within the City of Cape Town.  April 2007.  BVI Consulting Engineers for City of Cape Town, Water and Sanitation and Water Demand Management Departments

Law, I B.  2003.  Advanced re-use – from Windhoek to Singapore and beyond.  Water, May 2003, p 44.

WateReuse Foundation, 2004.  Best Practices for Developing Indirect Potable Reuse Projects: Phase 1 Report.  (Ref no 01-004-01).

# List of Appendices

Appendix A – Review of Previous Studies

Appendix B – List of best Practices and excerpts from the Water Re-use Foundation publication “Best Practices for Developing Indirect Potable Re-use Projects”

APPENDIX A

Review of Previous Studies

Reconciliation Study: Overview of Water Re-use potential from Wastewater Treatment Plants, and Treatment of Effluent to Potable Standards for Supply from the Faure Water Treatment Plant. June 2007

This was the most recent study undertaken and formed part of the Western Cape Water Supply System: Reconciliation Strategy Study.  The client was DWAF and City of Cape Town and the consultant was UWP Consulting.  They made use of the City’s Master Plan to provide an overview of the status of water re-use at the time.  A potential development option for direct potable re-use was conceptualised and costed.  This was compared against the cost of desalination.  It was found that the direct potable use option was viable and worthy of further consideration, but that indirect re-use or other means may be preferable due to social acceptability and the potential health risk.

Treated Effluent Re-use Strategy and Master Planning within the City of Cape Town. April 2007

This study was undertaken for the City of Cape Town (Water and Sanitation, Water Demand Management and Strategy) by BVI Consulting Engineers.  They undertook feasibility designs for large users within an economical distance of a wastewater treatment works (WWTWs), based on a unit cost of R2.50/kl.  They identified industrial and irrigation users, but did not look at supplying residential dwellings.  The bulk of the water re-use takes place in summer because most of the users are irrigators.

Draft Report: Re-use of Treated Wastewater within the City of Cape Town. December 2002

This is an unpublished report prepared for City of Cape Town by GLS.  The aim of the study was to get a better understanding of the potential for re-using treated wastewater for industrial/irrigation purposes within the CMA, and to refine the pre-feasibility wastewater re-use proposals contained in the IWRP Study.  They concluded that the most economical re-use opportunities would be those for which the treated wastewater effluent can be used directly with no additional post-treatment, and recommended that these opportunities be maximised.  This would involve ensuring that all WWTWs complied with the General Standard for Effluent.  They also recommended that a data base of large users and their quality requirements be set up, and then suitable post-treatment opportunities be investigated.

Integrated Water Resource Planning (IWRP) Study: Potential for the use of treated wastewater within the CMA. October 2001

This report was produced by Ninham Shand and Arcus Gibb for the City of Cape Town as part of the Integrated Water Resource Planning Study.  Three possible options were investigated, namely: reclamation to potable standard, irrigation exchange, and the potential for local use for irrigation and industry.  It was found that the cost of the first two options was too high for immediate implementation, but it was recommended that the potential for local use be pursued immediately.  (This was done in the form of the Master Plan implemented by the CCT).

Water Reclamation: A Strategic Guide. June 1999

This study was undertaken by Cape Wastewater Consultants for the Cape Metropolitan Council as part of the strategic evaluation of bulk wastewater.  Various development options were conceptualised and costed for direct potable and non-potable (commercial irrigation) re-use, and indirect non-potable re-use (aquifer recharge).  The main recommendation from this study was that the potential for re-use should be further investigated.  Shortly after this, the CCT was formed, with its associated complications as the six separate municipalities merged.  These recommendations were not a high priority and were not implemented at that stage.

Western Cape System Analysis – Utilisation of Less Conventional Water Sources. December 1992.

This study was undertaken by Ninham Shand for DWAF, and looked at groundwater (Cape Flats Aquifer – including recharge with effluent), sewage effluent, stormwater and desalination of seawater.  A development option was looked at whereby treated effluent from the purification works along the False Bay coast was reclaimed to potable standard and blended with water at the Faure Water Treatment Works.  The cost of this water was found to be high, but still lower than the desalination of seawater.  Another option was looked at whereby treated effluent would be exchanged for fresh water allocated to irrigators in the Stellenbosch area.  A further alternative was to use effluent directly for non-potable purposes, requiring a dual reticulation system.

An Investigation into the Reclamation of Sewage Effluent as a source of water supply to the Cape Metropolitan Area.  November 1979

This was undertaken for the purposes of a Masters Thesis for the University of Stellenbosch by JF Kapp. The re-use of sewage effluent in the Cape Metropolitan Area was investigated with a view to establish a time-relationship for the introduction of water re-use in the area.  The various forms of water re-use practiced globally were summarised, available technology was reviewed and health aspects considered.  A sample of public opinion with regard to water re-use was presented and the overall position of water supply and demand as well as of sewage works in the Cape Metropolitan Area was reviewed. The conclusion reached was that there was no immediate need to re-circulate reclaimed water, both from a demand and supply and an economic point of view.

APPENDIX B

List of Best Practices and Excerpts from the Water Re-use Foundation Publication :

Best Practices for Developing Indirect Potable Re-use Projects

| Top 10 | Best Practice Number | Description |  |
| --- | --- | --- | --- |
| - Value Based Strategy Best Practices |  |  |  |
| 1.1 Problem Being Solved |  |  |  |
| ** | #1 | Create a perception of improvement |  |
| * | #2 | Clearly articulate the problem |  |
|  | #3 | Have a meaningful supply planning criterion |  |
| 1.2 Alternatives to Indirect Potable Reuse |  |  |  |
| * | #4 | Evaluate alternatives to indirect potable reuse |  |
|  | #5 | Communicate all the benefits of indirect potable reuse |  |
|  | #6 | Express costs in meaningful terms |  |
| 1.3 Risks |  |  |  |
| * | #7 | Understand and avoid environmental justice issues |  |
|  | #8 | Understand and use track records |  |
|  | #9 | Break the source-quality connection |  |
|  | #10 | Articulate an ongoing water quality plan |  |
| - Identity of the Water Agency |  |  |  |
| * | #11 | Establish the Water Agency and investment as the sources of water quality |  |
|  | #12 | Rename the water quality |  |
| ## Communication Best Practices |  |  |  |
| ** | #13 | Communicate and collaborate about value |  |
|  | #14 | Inform, don’t educate |  |
| * | #15 | Practice good leadership |  |
|  | #16 | Start early |  |
| * | #17 | Identify and collaborate with key audiences |  |
| * | #18 | Embrace potential conflict and opposition |  |
|  | #19 | Constantly communicate |  |
|  | #20 | Keep communicating after the project is completed |  |
| * | #21 | Develop ongoing relationships with the media |  |
|  | #22 | Use general information / advertising campaigns appropriately |  |
|  | #23 | Don’t be defensive |  |
|  | #24 | Practice conflict resolution |  |
|  | #25 | Maintain credibility |  |